2.2.2	SUBMERSIBLE PUPMS FOR WASTEWATER OF THE MF TYPE

The distinguishing feature of the MF pump is a vortex impeller and a wide large size channel for the free passage of solid inclusions. This compact, resistant pump has a sealed, fully protected against leakage iron engine and a hydraulic part.

AREA OF APPLICATION

Submersible pumps of the MF type are used to solve problems related to water discharge inside and outside building

•	filling and emptying of the tanks used for drainage of flooded cellars and transport of storm water

•	suitable for installation in septic tanks as well as for pumping out wastewater, accumulating below the sewage and transporting them to the sewer in accordance with the EN 12056 standard

•	a special modification MF-VO equipped with a diverter knee, coupling for sleeves and mesh filter, which protects a pump from clogging (modification MF-VO can be supplied for MF 354, MF 504 and MF 804 models) was developed to drain building sites, ditches and other flooded places

•	Vortex impeller allows to pump liquids with gaseous and abrasive inclusions

•	compact pump design, ease mounting and a small size of the required settler provide fast and economical installation

•	efficient mounting

•	the maximum acceptable temperature for pumping fluid: 40°С for continuous operation and 60°С for short-term operation (no more than 5 minutes)

TECHNICAL SPECIFICATION

•	capacity – up to 58 m3/h

•	head – up to 16m

•	temperature – up to +40°С or up to +60°С during a short period (no more than 5 minutes)

OPTIONS

•	control of temperature and vibration

•	flushing of the neck ring

ADVANTAGES

•	versions with automatic level control function or without it

•	passage level of solid fractions from 20 to 60 mm

•	3 or 10m cable

•	compact form, providing ease mounting and small requirements to the well, allows a quick and cost-effective installation

•	pumps are available with some additions, including the shield, angled nozzle and hose coupling in order to be used in a construction site