| ИСХОДНИК | ПЕРЕВОД |
| --- | --- |
| 1. Опис. Таблетки від блідо-зеленого до зеленого кольору круглої форми з плоскою поверхнею зі скошеними краями і рискою. На поверхні таблеток допускається наявність вкраплень. 2. Ідентифікація. 2.1 На хроматограмі випробовуваного розчину, отриманій у розділі «Кількісне визначення», час утримування піку глімепіриду має співпадати з часом утримування піку глімепіриду на хроматограмі розчину порівняння з точністю 1 %. 2.2 Визначення проводять методом тонкошарової хроматографії, використовуючи хроматографічні пластинки Silica gel 60 F254 (фірма «Merck», Німеччина або аналогічної якості) на алюмінієвій основі, з товщиною шару 0,20 мм (ДФУ*, Ph.Eur.*, 2.2.27, 2.2.46). Випробовуваний розчин. 0,68 г порошку розтертих таблеток збовтують з 11 мл хлороформу Р протягом 7 хв, відстоюють, через 35 хв надосадову рідину фільтрують і використовують. Розчин порівняння. 10 мг стандартного зразка (СЗ) глімепіриду (EP CRS, або USP RS або робочого стандартного зразка (РСЗ) ПАТ «Фармак», або ф. «LGC», або аналогічної якості) розчиняють у хлороформі Р і доводять обєм розчину тим самим розчинником до 10 мл. | 1. Description. Pale green to green round flat-faced beveled-edge scored tablets with a flat surface with beveled edges and a streak. On the surface of the tablets, inclusions are allowed. 2. Identification 2.1 On the chromatogram of the test solution obtained in the " Quantitative determination " section, the retention time of the glimepiride peak should coincide with the retention time of the glimepiride peak on the solution chromatogram, with an accuracy of ± 1 %. 2.2 Determination is performed by a method of thin-layer chromatography using Silica gel 60 F254 chromatographic plates («Merck» firm, Germany or similar quality) on an aluminum basis, with a layer thickness of 0.20 mm (DFU , Ph.Eur. *, 2.2.27, 2.2 .46). The test solution. 0.68 g of powder from crumbled tablets are shaken with 11 ml of chloroform P for 7 min, stand, after 35 min the supernatant liquid is filtered and used. Reference solution. 10 mg of glimepiride standard sample (SS) (EP CRS, or USP RS or working standard sample (WSS) of PJSC "Pharmak" or F. "LGC" or similar quality) is dissolved in chloroform P and the volume of the solution is adjusted thereby solvent up to 10 ml. . |