| # Оригинал. http://petrowiki.org/Biological_monitoring Biodiversity Mapping. The Ormen Lange Field The Ormen Lange field is located at water depths of 800 to 1100 m, within the scar of the gigantic prehistoric Storegga Slide. The seabed in the area very rugged, with small softbottom troughs between peaks and ridges formed by slide blocks. The baseline survey and biodiversity mapping of the field therefore utilized both ROV survey with video transects, and traditional sediment sampling for macrofauna investigation. These complement each other, and were necessary in order to get a full overview of the benthic communities in the area. When petroleum activities move into new areas where the knowledge of biological processes and biodiversity is insufficient, there is a need for baseline survey to map the pre-activity situation. An equally important objective is to understand the environment in order to plan activities such that impacts are minimized. For petroleum activities on the Norwegian Continental Shelf (NCS) there is today a well-established procedure for how marine baseline and monitoring surveys should be carried out. This is part of the Activities Regulations1, which are part of the joint regulations relating to HSE on the NCS. It states that the operator shall carry out an environmental baseline survey to map the environmental status: - prior to exploration drilling in deep waters, - prior to exploration drilling in areas where the presence of particularly vulnerable environmental resources has been established or where the presence of such resources is likely, - prior to production drilling. The Activities regulations focus on sediment surveys, with investigation of macrofauna communities, sediment characteristics and concentration levels of hydrocarbons and metals. The methods prescribed are mainly directed towards monitoring of a homogenous and soft seabed where impacts may be traced as gradients from a point source. Ormen Lange is located in the Norwegian Sea, 100 km off the North West coast of Norway. The water depth makes this the deepest Norwegian offshore | Перевод Составление карты биологического разнообразия в области Омен Ланж Подводная область Омен Ланж находится на глубине 800 - 1100 м в пределах разлома и гигантского доисторического оползня Сторегга. Неровное морское дно покрыто небольшими впадинами и грядой подводных скал, сформированными оползневыми массивами. Характеристики дна и описание биологического разнообразия построены на исследованиях радиоуправляемых аппаратов ROV, сделавших видео съемку поперечных разрезов дна и традиционную выборку проб для изучения макрофауны. Оба исследования дополняют друг друга и были нужны для получения полного обзора по придонным сообществам данной территории. C продвижением нефтегазовой разведки в новые районы, где знание о биологических процессах и разнообразии видов недостаточно, возникает необходимость геодезического исследования и оценки ситуации к началу активности. Важной задачей является понимание окружающей среды. Это позволяет планировать мероприятия по вмешательству и сводить к минимуму последствия. Нефтегазовая деятельность на норвежском континентальном шельфе (NCS) сегодня осуществляется под регулярным мониторингом за изменениями подводной экосистемы. Это – часть правил компании и относится к международным нормативным актам по защите здоровья и окружающей среды HSE. Правила определяют проведение оператором базовых исследований окружающей среды для формирования экологического статуса: - до разведочного глубоководного бурения; - до разведочного бурения в районах, где выявлено или предполагается наличие уязвимых природных ресурсов; - до эксплуатационного бурения. Эксплуатационные нормативы строятся на данных ​​по обследованию осадочных пород, которые включают их характеристику, описание сообществ макрофауны, уровни концентрации углеводорода и металлов. Установленный порядок подводных разработок требует ведения контроля по влиянию на однородные и мягкие слои дна в пределах работы скважины. Омен Ланж расположен в Норвежском море, 100 км от северо-западного побережья Норвегии. Уровень воды делает этот оффшорный проект самым глубоководным на сегодняшний день. |
| --- | --- |