3D-PRINTED OBJECTS OUTGROW THEIR PRINTERS

by Niall Firth

1.3D printing may be set to change the world by letting us make all sorts of bespoke objects, but there's one little problem: the printers can only print items smaller than themselves. Until now, that is.

2.Skylar Tibbits at the Massachusetts Institute of Technology's Self-Assembly Lab and colleague Marcelo Coelho have come up with a way for standard 3D printers to print out large-scale objects. "It's challenging the notion that we always need a machine that's bigger than the thing it's printing," says Tibbits.

3.The approach, called Hyperform, converts the object to be printed into a single

long chain made from interlocking links. An algorithm works out how that chain can

be packed together into the smallest cube possible using a Hilbert curve – a fractalbased pattern that is the most efficient way of squeezing a single line into a small as space as possible. The resulting cube is small enough to be printed inside a standard printer.

Hand assembly

4.Once this cube is printed, the chain can be unravelled and assembled by hand

to create the desired object. That's possible because each link in the chain has notches

that allow it to bend only in a certain way. "You have to fold it by hand and click it

into place," says Tibbits. Hyperform won the "The Next Idea" prize at the Ars Electronica 2013 technology festival in Linz, Austria, earlier this month.

5.But printing cubes made of such densely packed chains was too much for most

of the consumer printers that Tibbits and his team tried. "We blew a lot of printers at

first," he says. So they teamed up with Formlabs who, after a successful Kickstarter

crowdfunding campaign, have just started shipping their Form 1 3D printer.

6.The Form 1 is capable of much higher resolution than standard consumer 3D

printers. Instead of printing out layer upon layer of plastic, it uses stereolithography,

in which a pool of liquid plastic is added to the base of the printer and a laser traces

out the pattern required, causing the liquid plastic to cure and solidify. The technique

can form layers just 25 microns thick, with details as small as 300 microns.

7.Hyperform has so far been used to create large structures such as a chandelier,

and Tibbits sees it as being perfect for producing large 3D-printed consumer

products. But the Form 1 printer uses resins which have limitations in terms of

strength. "There is a range of things that are largish that we can do right away," says

Tibbits. "But if you want to make large-scale furniture or buildings, there needs to be

an approach to make them stronger."

4D printing

8.Manually clicking each link into place isn't ideal either. That's where Tibbits'

other work in so-called 4D printing might help. 4D printing uses materials that are

3D-printed to produce an intermediate object which, when exposed to water, will

bend and twist itself into the final structure. "You can see how Hyperform and 4D

printing are pointing towards each other," he says.

9.Clément Moreau, CEO of French 3D printing firm Sculpteo, says projects like

Hyperform are shaping the future of 3D printing. "This is yet another example of how

3D printing is more of a flexible manufacturing process than injection moulding

because it constantly opens up new possibilities in terms of materials used and shapes

which can be printed."

Абзац 1. У цьому абзаці міститься опис проблеми – 3D принтери можуть друкувати предмети тільки менших за себе.

Абзац 2 розповідає про новий спосіб для стандартних 3D-принтерів друкувати великомасштабні об'єкти.

Абзац 3 характеризує алгоритм роботи Гіперформи та кінцевий результат.

Абзац 4 надає позитивну інформацію про друкований об*єкт, який потім можна скласти та розповідає про отриманні нагороди за цей підхід.

Абзац 5 інформує про негативний наслідок цього підходу та об*єднання з краудфандинговою кампанією для поставки 3d-принтера Form 1.

Абзац 6 розповідає про те, як працює 3d-принтер Form 1 і висвітлює його позитивні сторони.

Абзац 7 називає технічні труднощі, з якими не можливо створювати поки що великомасштабні меблі чи будівлі.

Абзац 8 характеризує 4D-принтер та його відмінність від 3d-принтера.

Абзац 9 робить висновок про успішне використання 3d-принтера.

Abstract

The article describes using 3D-printer in making outgrowing object, especially about approach called Hyperform. The algorithm of work and features of 3D-printer Form 1 are presented. The technical issues and advantages of this approach are covered.

1.3D printing may be set to change the world by letting us make all sorts of bespoke objects, but there's one little problem: the printers can only print items smaller than themselves. 2.Until now, that is.

3.Skylar Tibbits at the Massachusetts Institute of Technology's Self-Assembly Lab and colleague Marcelo Coelho have come up with a way for standard 3D printers to print out large-scale objects. 4."It's challenging the notion that we always need a machine that's bigger than the thing it's printing," says Tibbits.

5.The approach, called Hyperform, converts the object to be printed into a single long chain made from interlocking links. 6. An algorithm works out how that chain can

be packed together into the smallest cube possible using a Hilbert curve – a fractalbased pattern that is the most efficient way of squeezing a single line into a small as space as possible. 7. The resulting cube is small enough to be printed inside a standard printer.

Hand assembly

8.Once this cube is printed, the chain can be unravelled and assembled by hand

to create the desired object. 9. That's possible because each link in the chain has notches

that allow it to bend only in a certain way. 10. "You have to fold it by hand and click it

into place," says Tibbits. 11. Hyperform won the "The Next Idea" prize at the Ars Electronica 2013 technology festival in Linz, Austria, earlier this month.

12.But printing cubes made of such densely packed chains was too much for most of the consumer printers that Tibbits and his team tried. 13."We blew a lot of printers at first," he says. 14. So they teamed up with Formlabs who, after a successful Kickstarter crowdfunding campaign, have just started shipping their Form 1 3D printer.

15.The Form 1 is capable of much higher resolution than standard consumer 3D printers. 16. Instead of printing out layer upon layer of plastic, it uses stereolithography, in which a pool of liquid plastic is added to the base of the printer and a laser traces out the pattern required, causing the liquid plastic to cure and solidify. 17. The technique can form layers just 25 microns thick, with details as small as 300 microns.

18.Hyperform has so far been used to create large structures such as a chandelier, and Tibbits sees it as being perfect for producing large 3D-printed consumer products. 19.But the Form 1 printer uses resins which have limitations in terms of strength. 20. "There is a range of things that are largish that we can do right away," says Tibbits. 21. "But if you want to make large-scale furniture or buildings, there needs to be an approach to make them stronger."

4D printing

22.Manually clicking each link into place isn't ideal either. 23. That's where Tibbits' other work in so-called 4D printing might help. 24. 4D printing uses materials that are 3D-printed to produce an intermediate object which, when exposed to water, will bend and twist itself into the final structure. 25."You can see how Hyperform and 4D printing are pointing towards each other," he says.

26.Clément Moreau, CEO of French 3D printing firm Sculpteo, says projects like Hyperform are shaping the future of 3D printing. 27."This is yet another example of how 3D printing is more of a flexible manufacturing process than injection moulding because it constantly opens up new possibilities in terms of materials used and shapes which can be printed."

1. 3D printing may be set to change the world by letting us make all sorts of bespoke objects, but there's one little problem: the printers can only print items smaller than themselves. 
2. Until now, that is.
10. "You have to fold it by hand and click it into place," says Tibbits. 
11. Hyperform won the "The Next Idea" prize at the Ars Electronica 2013 technology festival in Linz, Austria, earlier this month.
13."We blew a lot of printers at first," he says. 
14. So they teamed up with Formlabs who, after a successful Kickstarter crowdfunding campaign, have just started shipping their Form 1 3D printer.

18. Hyperform has so far been used to create large structures such as a chandelier, and Tibbits sees it as being perfect for producing large 3D-printed consumer products.

22. Manually clicking each link into place isn't ideal either.

25."You can see how Hyperform and 4D printing are pointing towards each other," he says.

26. Clément Moreau, CEO of French 3D printing firm Sculpteo, says projects like Hyperform are shaping the future of 3D printing.