No. 1	Location of SPP: Biloghirya territorial community of Khmelnytskyi region

Gulivtsi Lyceum

Description: Rooftop SPP

Power of the solar field: 15 kW

Inverter power: 12 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 27 | 8,282.90 ₴ | 223,638.30 ₴ |
| 2. Electricity inverter | Solis S5-GR3P12K | 1 | 54,975.00 ₴ | 54,975.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 14,843.25 ₴ | 14,843.25 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 120 | 40.00 ₴ | 4,800.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,800.00 ₴ | 3,800.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 6. Consumables |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 7. Transportation |  | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| 8. Assembly works |  | 1 | 42,000.00 ₴ | 42,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 394,356.00 ₴ |

CALCULATION

CAPEX

SPP 15 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 15 KW.

3,300

3,000

2,700

2,400

2 100

1,800

1,500

1,200

900

600

300

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

3,200

2,868

3,000 3,000

2,500 2,500 2,500

2,660

1 828

2,2122,274 2,236

2 306

1 932

2,529

1 928

1 313

1 681 1 391

761

913

453

434

340

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec

SPP generation, kW/h	 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 453 | 2500 | 2047 |
| February | 761 | 2500 | 1739 |
| March | 1313 | 2500 | 1187 |
| April | 1828 | 2868 | 1040 |
| May | 2212 | 2660 | 448 |
| June | 2274 | 2306 | 32 |
| July | 2236 | 1932 | - |
| August | 1928 | 1681 | - |
| September | 1391 | 2529 | 1138 |
| October | 913 | 3000 | 2087 |
| November | 434 | 3000 | 2566 |
| December | 340 | 3200 | 2860 |
| TOTAL: | 16083 | 30676 | 15144 |

| CAPEX (SPP cost) | 394,356.00 | UAH |
| --- | --- | --- |
| Power of SPP | 15 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 16,083 | kW/h |
| Consumption electricity an object | 30,676 | kW/h |
| Online purchase | 15 144 | kW/h |
| Coverage of consumption by own SPP | 15,532 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 551 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 90,864 | UAH |
| Costs on service by 1 year | 8,000 | UAH |
| The cost of projected energy consumption from own SPP | 93 192 | UAH |
| Cost consumption electricity without installed SPP | 184,056 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 85 192 | UAH |
| Payback of the project | 4.6 | years |
| The cost of generated electricity after payback | 0.50 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 15 KW.

No. 2	Location of SPP: Biloghirya territorial community of Khmelnytskyi region

Denisiv Gymnasium

Description: Rooftop SPP

Power of the solar field: 8 kW

Inverter power: 6 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 15 | 7,586.55 ₴ | 110,349.82 ₴ |
| 2. Electricity inverter | Solis S5-GR3P6K | 1 | 45,812.50 ₴ | 45,812.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 7,996.36 ₴ | 7,996.36 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 80 | 40.00 ₴ | 3,200.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,000.00 ₴ | 2,000.00 ₴ |
| 6. Consumables |  | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 7. Transportation |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 8. Assembly works |  | 1 | 22,400.00 ₴ | 22,400.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 239,058.13 ₴ |

CALCULATION

CAPEX

SPP 8 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 8 KW.

1,350

1,200

1,050

900

750

600

450

300

150

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

1 2081 240 1 224

1 104

992

900

970

970

720

9501,000

850

784

750

850

504

416

500

248

256

216

200	150	150

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec

SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 248 | 900 | 652 |
| February | 416 | 970 | 554 |
| March | 720 | 970 | 250 |
| April | 992 | 850 | - |
| May | 1 208 | 500 | - |
| June | 1,240 | 200 | - |
| July | 1 224 | 150 | - |
| August | 1 104 | 150 | - |
| September | 784 | 750 | - |
| October | 504 | 850 | 346 |
| November | 256 | 950 | 694 |
| December | 216 | 1000 | 784 |
| TOTAL: | 8,912 | 8240 | 3280 |

| CAPEX (SPP cost) | 239,058.13 | UAH |
| --- | --- | --- |
| Power SPP | 8 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 8,912 | kW/h |
| Consumption electricity an object | 8,240 | kW/h |
| Online purchase | 3,280 | kW/h |
| Coverage of consumption by own SPP | 4,960 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 3,952 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 19,680 | UAH |
| Costs on service by 1 year | 5,000 | UAH |
| The cost of projected energy consumption from own SPP | 29,760 | UAH |
| Cost consumption electricity without installed SPP | 49,440 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 24,760 | UAH |
| Payback of the project | 9.7 | years |
| The cost of generated electricity after payback | 0.56 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 8 KW.

No. 3	Location of SPP: Biloghirya territorial community of Khmelnytskyi region

Kvitneve Lyceum

Description: Rooftop SPP

Power of the solar field: 8 kW

Inverter power: 6 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 15 | 7,586.55 ₴ | 110,349.82 ₴ |
| 2. Electricity inverter | Solis S5-GR3P6K | 1 | 45,812.50 ₴ | 45,812.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 7,996.36 ₴ | 7,996.36 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 80 | 40.00 ₴ | 3,200.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,000.00 ₴ | 2,000.00 ₴ |
| 6. Consumables |  | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 7. Transportation |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 8. Assembly works |  | 1 | 22,400.00 ₴ | 22,400.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 239,058.13 ₴ |

CALCULATION

CAPEX

SPP 8 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 8 KW.

1,800

1,600

1,400

1,200

1,000

800

600

400

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

1 728 1 702

1 2081 240 1 224

1 104

1919224

800

850

720

876

784

416	500

504

500

700

248	400

600

256

216

100

200

0

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec

SPP generation, kW/h	 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 248 | 800 | 552 |
| February | 416 | 850 | 434 |
| March | 720 | 500 | - |
| April | 992 | 1124 | 132 |
| May | 1 208 | 1728 | 520 |
| June | 1,240 | 1702 | 462 |
| July | 1 224 | 876 | - |
| August | 1 104 | 100 | - |
| September | 784 | 400 | - |
| October | 504 | 500 | - |
| November | 256 | 600 | 344 |
| December | 216 | 700 | 484 |
| TOTAL: | 8,912 | 9880 | 2928 |

| CAPEX (SPP cost) | 239,058.13 | UAH |
| --- | --- | --- |
| Power of SPP | 8 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 8,912 | kW/h |
| Consumption electricity an object | 9,880 | kW/h |
| Online purchase | 2,928 | kW/h |
| Coverage of consumption by own SPP | 6,952 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 1 960 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 17,568 | UAH |
| Costs on service by 1 year | 5,000 | UAH |
| The cost of projected energy consumption from own SPP | 41,712 | UAH |
| Cost consumption electricity without installed SPP | 59,280 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 36,712 | UAH |
| Payback of the project | 6.5 | years |
| The cost of generated electricity after payback | 0.56 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 8 KW.

No. 4	Location of SPP: Biloghirya territorial community of Khmelnytskyi region

Pererosliv Lyceum

Description: Rooftop SPP

Power of the solar field: 8 kW

Inverter power: 6 kW

Biloghirya territorial community

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 15 | 7,586.55 ₴ | 110,349.82 ₴ |
| 2. Electricity inverter | Solis S5-GR3P6K | 1 | 45,812.50 ₴ | 45,812.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 7,996.36 ₴ | 7,996.36 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 80 | 40.00 ₴ | 3,200.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,000.00 ₴ | 2,000.00 ₴ |
| 6. Consumables |  | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 7. Transportation |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 8. Assembly works |  | 1 | 22,400.00 ₴ | 22,400.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 239,058.13 ₴ |

CALCULATION

CAPEX

SPP 8 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 8 KW.

1,650

1,500

1,350

1,200

1,050

900

750

600

450

300

150

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

1,450

1,530

1,600

1,450

1 2081 240 1 224

1 104

1,400

1,300

992

1,050

720

920

784

730

416

600

504

248

256

216

150	150

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 248 | 1450 | 1 202 |
| February | 416 | 1530 | 1 114 |
| March | 720 | 1450 | 730 |
| April | 992 | 1050 | 58 |
| May | 1 208 | 920 | - |
| June | 1,240 | 730 | - |
| July | 1 224 | 150 | - |
| August | 1 104 | 150 | - |
| September | 784 | 600 | - |
| October | 504 | 1300 | 796 |
| November | 256 | 1400 | 1 144 |
| December | 216 | 1600 | 1 384 |
| TOTAL: | 8,912 | 12330 | 6428 |

| CAPEX (SPP cost) | 239,058.13 | UAH |
| --- | --- | --- |
| Power of SPP | 8 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 8,912 | kW/h |
| Consumption electricity an object | 12,330 | kW/h |
| Online purchase | 6,428 | kW/h |
| Coverage of consumption by own SPP | 5,902 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 3,010 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 38,568 | UAH |
| Costs on service by 1 year | 5,000 | UAH |
| The cost of projected energy consumption from own SPP | 35,412 | UAH |
| Cost consumption electricity without installed SPP | 73,980 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 30,412 | UAH |
| Payback of the project | 7.9 | years |
| The cost of generated electricity after payback | 0.56 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 8 KW.

No. 5	Location of SPP: Biloghirya territorial community of Khmelnytskyi region

Sosnivka Gymnasium

Description: Rooftop SPP

Power of the solar field: 8 kW

Inverter power: 6 kW

CALCULATION

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 15 | 7,586.55 ₴ | 110,349.82 ₴ |
| 2. Electricity inverter | Solis S5-GR3P6K | 1 | 45,812.50 ₴ | 45,812.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 7,996.36 ₴ | 7,996.36 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 80 | 40.00 ₴ | 3,200.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,000.00 ₴ | 2,000.00 ₴ |
| 6. Consumables |  | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 7. Transportation |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 8. Assembly works |  | 1 | 22,400.00 ₴ | 22,400.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 239,058.13 ₴ |

CAPEX

SPP 8 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 8 KW.

1,650

1,500

1,350

1,200

1,050

900

750

600

450

300

150

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

1,600

1,500 1,500

1 2081 240 1 224

1 335

1 104

992

950	943

782106

784

747

416

504

544

248

312

256

216

200

200

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 248 | 950 | 702 |
| February | 416 | 943 | 527 |
| March | 720 | 816 | 96 |
| April | 992 | 544 | - |
| May | 1 208 | 747 | - |
| June | 1,240 | 200 | - |
| July | 1 224 | 312 | - |
| August | 1 104 | 200 | - |
| September | 784 | 1335 | 551 |
| October | 504 | 1500 | 996 |
| November | 256 | 1500 | 1 244 |
| December | 216 | 1600 | 1 384 |
| TOTAL: | 8,912 | 10647 | 5500 |

| CAPEX (SPP cost) | 239,058.13 | UAH |
| --- | --- | --- |
| Power of SPP | 8 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 8,912 | kW/h |
| Consumption electricity an object | 10,647 | kW/h |
| Online purchase | 5,500 | kW/h |
| Coverage of consumption by own SPP | 5 147 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 3,765 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 33,000 | UAH |
| Costs on service by 1 year | 5,000 | UAH |
| The cost of projected energy consumption from own SPP | 30,882 | UAH |
| Cost consumption electricity without installed SPP | 63,882 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 25,882 | UAH |
| Payback of the project | 9.2 | years |
| The cost of generated electricity after payback | 0.56 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 8 KW.

No. 6	Location of SPP: Biloghirya territorial community of Khmelnytskyi region

Biloghirya support institution of the general secondary education named after I.O. Weaver

Description: Rooftop SPP

Power of the solar field: 12 kW

Inverter power: 10 kW

CALCULATION

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 22 | 7,586.55 ₴ | 165,524.73 ₴ |
| 2. Electricity inverter | Solis S5-GR3P10K | 1 | 53,142.50 ₴ | 53,142.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 11,994.55 ₴ | 11,994.55 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 100 | 40.00 ₴ | 4,000.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,500.00 ₴ | 3,500.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,600.00 ₴ | 3,600.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,300.00 ₴ | 2,300.00 ₴ |
| 6. Consumables |  | 1 | 2,800.00 ₴ | 2,800.00 ₴ |
| 7. Transportation |  | 1 | 4,500.00 ₴ | 4,500.00 ₴ |
| 8. Assembly works |  | 1 | 33,600.00 ₴ | 33,600.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 320,761.22 ₴ |

CAPEX

SPP 12 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 12 KW.

5,800

5,220

4,640

4,060

3 480

2,900

2,320

1 740

1 160

580

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

5,700

5 104

5,350

4,7504,800

4 114

2,986

2,729

2 211

1 488

1,812 1,860 1,836 1,656

2,050

2 043

1,080

1,350

1 176

372

624

756

384

324

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 372 | 2 211 | 1 839 |
| February | 624 | 2,986 | 2 362 |
| March | 1,080 | 2,729 | 1 649 |
| April | 1 488 | 5 104 | 3,616 |
| May | 1 812 | 4 114 | 2 302 |
| June | 1 860 | 2,050 | 190 |
| July | 1 836 | 1,350 | - |
| August | 1 656 | 2 043 | 387 |
| September | 1 176 | 4,750 | 3,574 |
| October | 756 | 4,800 | 4,044 |
| November | 384 | 5,350 | 4,966 |
| December | 324 | 5,700 | 5,376 |
| TOTAL: | 13,368 | 43 187 | 30,305 |

| CAPEX (SPP cost) | 320,761.22 | UAH |
| --- | --- | --- |
| Power of SPP | 12 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 13,368 | kW/h |
| Consumption electricity an object | 43 187 | kW/h |
| Online purchase | 30,305 | kW/h |
| Coverage of consumption by own SPP | 12,882 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 486 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 181,830 | UAH |
| Costs on service by 1 year | 7,000 | UAH |
| The cost of projected energy consumption from own SPP | 77,292 | UAH |
| Cost consumption electricity without installed SPP | 259 122 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 70,292 | UAH |
| Payback of the project | 4.6 | years |
| The cost of generated electricity after payback | 0.52 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 12 KW.

No7	Location of SPP: Medzhybizh territorial community

Trebuhivtsi secondary school of grades I-III

Description: Rooftop SPP

Power of the solar field: 8 kW

Inverter power: 6 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 15 | 7,586.55 ₴ | 110,349.82 ₴ |
| 2. Electricity inverter | Solis S5-GR3P6K | 1 | 45,812.50 ₴ | 45,812.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 7,996.36 ₴ | 7,996.36 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 80 | 40.00 ₴ | 3,200.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,000.00 ₴ | 2,000.00 ₴ |
| 6. Consumables |  | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 7. Transportation |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 8. Assembly works |  | 1 | 22,400.00 ₴ | 22,400.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 239,058.13 ₴ |

CALCULATION

CAPEX

SPP 8 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 8 KW.

3,800

3,420

3,040

2,660

2,280

1,900

1,520

1 140

760

380

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

3,780

3 150

2 335

2,050

1 843

992

7201,050

1,208 1,240 1,224 1,104

900

700

300	200

1 447

1718640

504

248

416

256

216

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 248 | 2,050 | 1 802 |
| February | 416 | 3,780 | 3 364 |
| March | 720 | 2 335 | 1,615 |
| April | 992 | 1,050 | 58 |
| May | 1 208 | 900 | - |
| June | 1,240 | 700 | - |
| July | 1 224 | 300 | - |
| August | 1 104 | 200 | - |
| September | 784 | 1 160 | 376 |
| October | 504 | 1 447 | 943 |
| November | 256 | 1 843 | 1 587 |
| December | 216 | 3 150 | 2,934 |
| TOTAL: | 8,912 | 18,915 | 12,679 |

| CAPEX (SPP cost) | 239,058.13 | UAH |
| --- | --- | --- |
| Power of SPP | 8 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 8,912 | kW/h |
| Consumption electricity an object | 18,915 | kW/h |
| Online purchase | 12,679 | kW/h |
| Coverage of consumption by own SPP | 6,236 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 2,676 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 76,074 | UAH |
| Costs on service by 1 year | 5,000 | UAH |
| The cost of projected energy consumption from own SPP | 37,416 | UAH |
| Cost consumption electricity without installed SPP | 113,490 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 32,416 | UAH |
| Payback of the project | 7.4 | years |
| The cost of generated electricity after payback | 0.56 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 8 KW.

No. 8	Location of SPP: Stara Ushytsia territorial community

Stara Ushytsia Local Government Building

Description: Rooftop SPP

Power of the solar field:15 kW

Inverter power:12 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 27 | 8,282.90 ₴ | 223,638.30 ₴ |
| 2. Electricity inverter | Solis S5-GR3P12K | 1 | 54,975.00 ₴ | 54,975.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 14,843.25 ₴ | 14,843.25 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 120 | 40.00 ₴ | 4,800.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,800.00 ₴ | 3,800.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 6. Consumables |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 7. Transportation |  | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| 8. Assembly works |  | 1 | 42,000.00 ₴ | 42,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 394,356.00 ₴ |

CALCULATION

CAPEX

SPP 15 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 15 KW.

10,000

9,000

8,000

7,000

6,000

5,000

4,000

3,000

2,000

1,000

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

9,885

8,921

8,614

6,040

4,626

1 313

3,697

1 8282,212 2,274 2,236 1,928

1 391 2 060

453

761

1 141 1 183

1,050

1 231

913

956

434

340

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 453 | 8,921 | 8 468 |
| February | 761 | 9,885 | 9 124 |
| March | 1 313 | 8,614 | 7 301 |
| April | 1 828 | 3,697 | 1 869 |
| May | 2,212 | 1 141 | - |
| June | 2,274 | 1 183 | - |
| July | 2 236 | 1,050 | - |
| August | 1 928 | 1 231 | - |
| September | 1 391 | 956 | - |
| October | 913 | 2,060 | 1 147 |
| November | 434 | 4,626 | 4 192 |
| December | 340 | 6,040 | 5,700 |
| TOTAL: | 16,083 | 49 404 | 37,801 |

| CAPEX (SPP cost) | 394,356.00 | UAH |
| --- | --- | --- |
| Power of SPP | 15 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 16,083 | kW/h |
| Consumption electricity an object | 49 404 | kW/h |
| Online purchase | 37,801 | kW/h |
| Coverage of consumption by own SPP | 11,603 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 4,480 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 226,806 | UAH |
| Costs on service by 1 year | 8,000 | UAH |
| The cost of projected energy consumption from own SPP | 69,618 | UAH |
| Cost consumption electricity without installed SPP | 296 424 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 61,618 | UAH |
| Payback of the project | 6.4 | years |
| The cost of generated electricity after payback | 0.50 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 15 KW.

No. 9	Location of SPP: Stara Ushytsia territorial community

Stara Ushytsia Lyceum

Description: Rooftop SPP

Power of the solar field:8 kW

Inverter power:6 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 15 | 7,586.55 ₴ | 110,349.82 ₴ |
| 2. Electricity inverter | Solis S5-GR3P6K | 1 | 45,812.50 ₴ | 45,812.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 7,996.36 ₴ | 7,996.36 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 80 | 40.00 ₴ | 3,200.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,000.00 ₴ | 2,000.00 ₴ |
| 6. Consumables |  | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 7. Transportation |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 8. Assembly works |  | 1 | 22,400.00 ₴ | 22,400.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 239,058.13 ₴ |

CALCULATION

CAPEX

SPP 8 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 8 KW.

59,220

48,222

40 163

41,847

23,650

248

January

416

February

9,001

720 1939629 1 230386 1 124403 1 820241 41404 788441	504

256	216

Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec

60,000

54,000

48,000

42,000

36,000

30,000

24,000

18,000

12,000

6,000

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

SPP generation, kW/h	 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 248 | 59,220 | 58,972 |
| February | 416 | 40 163 | 39,747 |
| March | 720 | 23,650 | 22,930 |
| April | 992 | 1 369 | 377 |
| May | 1 208 | 1 336 | 128 |
| June | 1,240 | 1 133 | - |
| July | 1 224 | 80 | - |
| August | 1 104 | 44 | - |
| September | 784 | 841 | 57 |
| October | 504 | 9,001 | 8,497 |
| November | 256 | 41,847 | 41,591 |
| December | 216 | 48,438 | 48,222 |
| TOTAL: | 8,912 | 227 122 | 220,521 |

| CAPEX (SPP cost) | 239,058.13 | UAH |
| --- | --- | --- |
| Power of SPP | 8 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 8,912 | kW/h |
| Consumption electricity an object | 227 122 | kW/h |
| Online purchase | 220,521 | kW/h |
| Coverage of consumption by own SPP | 6,601 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 2 311 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 1,323,126 | UAH |
| Costs on service by 1 year | 5,000 | UAH |
| The cost of projected energy consumption from own SPP | 39,606 | UAH |
| Cost consumption electricity without installed SPP | 1,362,732 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 34,606 | UAH |
| Payback of the project | 6.9 | years |
| The cost of generated electricity after payback | 0.56 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 8 KW.

No10

Location of SPP: Berezdiv territorial community

Berezdiv Lyceum

Description: Rooftop SPP

Power of the solar field: 15 kW

Inverter power: 12 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 27 | 8,282.90 ₴ | 223,638.30 ₴ |
| 2. Electricity inverter | Solis S5-GR3P12K | 1 | 54,975.00 ₴ | 54,975.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 14,843.25 ₴ | 14,843.25 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 120 | 40.00 ₴ | 4,800.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,800.00 ₴ | 3,800.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 6. Consumables |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 7. Transportation |  | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| 8. Assembly works |  | 1 | 42,000.00 ₴ | 42,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 394,356.00 ₴ |

CALCULATION

CAPEX

SPP 15 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 15 KW.

8,200

7,380

6,560

5,740

4,920

4 100

3,280

2,460

1,640

820

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

1 828

2,212 2,274 2,236 1,928

1 313

1 391

913

453	761

434

340

4,000	8000	5000	4000	3000	1000	2000	2000	3000	6000	6000	8000

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 453 | 4,000 | 3 547 |
| February | 761 | 8,000 | 7,239 |
| March | 1 313 | 5,000 | 3,687 |
| April | 1 828 | 4,000 | 2 172 |
| May | 2,212 | 3,000 | 788 |
| June | 2,274 | 1,000 | - |
| July | 2 236 | 2,000 | - |
| August | 1 928 | 2,000 | 72 |
| September | 1 391 | 3,000 | 1 609 |
| October | 913 | 6,000 | 5,087 |
| November | 434 | 6,000 | 5,566 |
| December | 340 | 8,000 | 7,660 |
| TOTAL: | 16,083 | 52,000 | 37,427 |

| CAPEX (SPP cost) | 394,356.00 | UAH |
| --- | --- | --- |
| Power of SPP | 15 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 16,083 | kW/h |
| Consumption electricity an object | 52,000 | kW/h |
| Online purchase | 37,427 | kW/h |
| Coverage of consumption by own SPP | 14,573 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 1,510 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 224,562 | UAH |
| Costs on service by 1 year | 8,000 | UAH |
| The cost of projected energy consumption from own SPP | 87 438 | UAH |
| Cost consumption electricity without installed SPP | 312,000 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 79,438 | UAH |
| Payback of the project | 5.0 | years |
| The cost of generated electricity after payback | 0.50 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 15 KW.

No11

Location of SPP: Berezdiv territorial community

Malopravutyn Lyceum

Description: Rooftop SPP

Power of the solar field: 8 kW

Inverter power: 6 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 15 | 7,586.55 ₴ | 110,349.82 ₴ |
| 2. Electricity inverter | Solis S5-GR3P6K | 1 | 45,812.50 ₴ | 45,812.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 7,996.36 ₴ | 7,996.36 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 80 | 40.00 ₴ | 3,200.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,000.00 ₴ | 2,000.00 ₴ |
| 6. Consumables |  | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 7. Transportation |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 8. Assembly works |  | 1 | 22,400.00 ₴ | 22,400.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 239,058.13 ₴ |

CALCULATION

CAPEX

SPP 8 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 8 KW.

3,200

2,880

2,560

2,240

1 920

1,600

1,280

960

640

320

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

1 2081 240 1 224

992

1 104

720

784

416	504

248	256

216

1,000 2,000 2,000 2,000 1,000 1,000 1,000 2,000 2,000 2,000 3,000 3,000

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 248 | 1,000 | 752 |
| February | 416 | 2,000 | 1,584 |
| March | 720 | 2,000 | 1,280 |
| April | 992 | 2,000 | 1 008 |
| May | 1 208 | 1,000 | - |
| June | 1,240 | 1,000 | - |
| July | 1 224 | 1,000 | - |
| August | 1 104 | 2,000 | 896 |
| September | 784 | 2,000 | 1 216 |
| October | 504 | 2,000 | 1 496 |
| November | 256 | 3,000 | 2,744 |
| December | 216 | 3,000 | 2,784 |
| TOTAL: | 8,912 | 22,000 | 13,760 |

| CAPEX (SPP cost) | 239,058.13 | UAH |
| --- | --- | --- |
| Power of SPP | 8 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 8,912 | kW/h |
| Consumption electricity an object | 22,000 | kW/h |
| Online purchase | 13,760 | kW/h |
| Coverage of consumption by own SPP | 8,240 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 672 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 82,560 | UAH |
| Costs on service by 1 year | 5,000 | UAH |
| The cost of projected energy consumption from own SPP | 49,440 | UAH |
| Cost consumption electricity without installed SPP | 132,000 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 44,440 | UAH |
| Payback of the project | 5.4 | years |
| The cost of generated electricity after payback | 0.56 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 8 KW.

No12

Location of SPP: Berezdiv territorial community

Berezdiv AZPSM

Description: Rooftop SPP

Power of the solar field: 8 kW

Inverter power: 6 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 15 | 7,586.55 ₴ | 110,349.82 ₴ |
| 2. Electricity inverter | Solis S5-GR3P6K | 1 | 45,812.50 ₴ | 45,812.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 7,996.36 ₴ | 7,996.36 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 80 | 40.00 ₴ | 3,200.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,000.00 ₴ | 2,000.00 ₴ |
| 6. Consumables |  | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 7. Transportation |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 8. Assembly works |  | 1 | 22,400.00 ₴ | 22,400.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 239,058.13 ₴ |

CALCULATION

CAPEX

SPP 8 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 8 KW.

2,600

2,275

1 950

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

1 2081 240 1 224

992

1 104

720

784

416

504

248

256

216

1,500	2000	2000	2000	1500	1500	1500	1000	2000	2000	2500	2500

1,625

1,300

975

650

325

0

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 248 | 1,500 | 1 252 |
| February | 416 | 2,000 | 1,584 |
| March | 720 | 2,000 | 1,280 |
| April | 992 | 2,000 | 1 008 |
| May | 1 208 | 1,500 | 292 |
| June | 1,240 | 1,500 | 260 |
| July | 1 224 | 1,500 | 276 |
| August | 1 104 | 1,000 | - |
| September | 784 | 2,000 | 1 216 |
| October | 504 | 2,000 | 1 496 |
| November | 256 | 2,500 | 2 244 |
| December | 216 | 2,500 | 2,284 |
| TOTAL: | 8,912 | 22,000 | 13 192 |

| CAPEX (SPP cost) | 239,058.13 | UAH |
| --- | --- | --- |
| Power of SPP | 8 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 8,912 | kW/h |
| Consumption electricity an object | 22,000 | kW/h |
| Online purchase | 13 192 | kW/h |
| Coverage of consumption by own SPP | 8,808 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 104 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 79 152 | UAH |
| Costs on service by 1 year | 5,000 | UAH |
| The cost of projected energy consumption from own SPP | 52,848 | UAH |
| Cost consumption electricity without installed SPP | 132,000 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 47,848 | UAH |
| Payback of the project | 5.0 | years |
| The cost of generated electricity after payback | 0.56 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 8 KW.

No13

Location of SPP: Dunayevetsi territorial community

SE "Miskwodokanal"

Description: Rooftop SPP

Power of the solar field: 30 kW

Inverter power: 30 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 55 | 8,466.15 ₴ | 461,790.00 ₴ |
| 2. Electricity inverter | SolisS5-GC30K | 1 | 76,965.00 ₴ | 76,965.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 39,981.82 ₴ | 39,981.82 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X25 | 10 | 800.00 ₴ | 800.00 ₴ |
| cable for solar panels | KBE | 200 | 40.00 ₴ | 8,000.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 5,500.00 ₴ | 5,500.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 3,500.00 ₴ | 3,500.00 ₴ |
| 6. Consumables |  | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| 7. Transportation |  | 1 | 10,000.00 ₴ | 10,000.00 ₴ |
| 8. Assembly works |  | 1 | 84,000.00 ₴ | 84,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 734,736.27 ₴ |

CALCULATION

CAPEX

SPP 30 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 30 KW.

81,300

73,717

68,804

72 439 71 10272 133

65,737

67,39068,243

63 463 63 630

61,681

930

January

1,560 2,700 3,7204,530 4,650 4,590 4,140 2,940 1,890

February

Mar

Ap

May

June

July

Aug

Sept	 Oct

960	810

Nov	Dec

82,000

73,800

65,600

57,400

49,200

41,000

32,800

24,600

16,400

8,200

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

SPP generation, kW/h	 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 930 | 81,300 | 80,370 |
| February | 1,560 | 73,717 | 72 157 |
| March | 2,700 | 68,804 | 66 104 |
| April | 3,720 | 65,737 | 62,017 |
| May | 4,530 | 72,439 | 67,909 |
| June | 4,650 | 71 102 | 66,452 |
| July | 4,590 | 72 133 | 67,543 |
| August | 4 140 | 63 463 | 59,323 |
| September | 2,940 | 63,630 | 60,690 |
| October | 1 890 | 67,390 | 65,500 |
| November | 960 | 68,243 | 67,283 |
| December | 810 | 61,681 | 60,871 |
| TOTAL: | 33,420 | 829,639 | 796 219 |

| CAPEX (SPP cost) | 734,736.27 | UAH |
| --- | --- | --- |
| Power of SPP | 30 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 33,420 | kW/h |
| Consumption electricity an object | 829,639 | kW/h |
| Online purchase | 796 219 | kW/h |
| Coverage of consumption by own SPP | 33,420 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 0 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 4,777,314 | UAH |
| Costs on service by 1 year | 17,000 | UAH |
| The cost of projected energy consumption from own SPP | 200,520 | UAH |
| Cost consumption electricity without installed SPP | 4,977,834 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 183,520 | UAH |
| Payback of the project | 4.0 | Years |
| The cost of generated electricity after payback | 0.51 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 30 KW.

No14

Location of SPP : Zinkiv territorial community

Zinkiv Lyceum

Description: Rooftop SPP

Power of the solar field:12 kW

Inverter power:10 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 22 | 8,282.90 ₴ | 180,717.82 ₴ |
| 2. Electricity inverter | Solis S5-GR3P10K | 1 | 53,142.50 ₴ | 53,142.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 11,994.55 ₴ | 11,994.55 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 100 | 40.00 ₴ | 4,000.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,500.00 ₴ | 3,500.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,600.00 ₴ | 3,600.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,300.00 ₴ | 2,300.00 ₴ |
| 6. Consumables |  | 1 | 2,800.00 ₴ | 2,800.00 ₴ |
| 7. Transportation |  | 1 | 4,500.00 ₴ | 4,500.00 ₴ |
| 8. Assembly works |  | 1 | 33,600.00 ₴ | 33,600.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 335,954.31 ₴ |

CALCULATION

CAPEX

SPP 12 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 12 KW.

5,800

5,220

4,640

4,060

3 480

2,900

2,320

1 740

1 160

580

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

5,700

5 104

5,350

4,7504,800

4 114

2,986

2,729

2 211

1 488

1,812 1,860 1,836 1,656

2,050

2 043

1,080

1,350

1 176

372

624

756

384

324

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 372 | 2,200 | 1 828 |
| February | 624 | 6,200 | 5,576 |
| March | 1,080 | 4,050 | 2,970 |
| April | 1 488 | 1,300 | - |
| May | 1 812 | 3,080 | 1 268 |
| June | 1 860 | 1,590 | - |
| July | 1 836 | 190 | - |
| August | 1 656 | 340 | - |
| September | 1 176 | 2,760 | 1,584 |
| October | 756 | 2,460 | 1 704 |
| November | 384 | 4,200 | 3,816 |
| December | 324 | 5,530 | 5,206 |
| TOTAL: | 13,368 | 33,900 | 23952 |

| CAPEX (SPP cost) | 335,954.31 | UAH |
| --- | --- | --- |
| Power of SPP | 12 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 13,368 | kW/h |
| Consumption electricity an object | 33,900 | kW/h |
| Online purchase | 23,952 | kW/h |
| Coverage of consumption by own SPP | 9,948 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 3,420 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 143,712 | UAH |
| Costs on service by 1 year | 7,000 | UAH |
| The cost of projected energy consumption from own SPP | 59,688 | UAH |
| Cost consumption electricity without installed SPP | 203,400 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 52,688 | UAH |
| Payback of the project | 6.4 | years |
| The cost of generated electricity after payback | 0.52 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 12 KW.

No15

Location of SPP : Zinkiv territorial community

Zinkiv Center for the Provision of Administrative Services

Description: Rooftop SPP

Power of the solar field:12 kW

Inverter power:10 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 22 | 8,282.90 ₴ | 180,717.82 ₴ |
| 2. Electricity inverter | Solis S5-GR3P10K | 1 | 53,142.50 ₴ | 53,142.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 11,994.55 ₴ | 11,994.55 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 100 | 40.00 ₴ | 4,000.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,500.00 ₴ | 3,500.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,600.00 ₴ | 3,600.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,300.00 ₴ | 2,300.00 ₴ |
| 6. Consumables |  | 1 | 2,800.00 ₴ | 2,800.00 ₴ |
| 7. Transportation |  | 1 | 4,500.00 ₴ | 4,500.00 ₴ |
| 8. Assembly works |  | 1 | 33,600.00 ₴ | 33,600.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 335,954.31 ₴ |

CALCULATION

CAPEX

SPP 12 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 12 KW.

5,800

5,220

4,640

4,060

3 480

2,900

2,320

1 740

1 160

580

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

5,700

5 104

5,350

4,7504,800

4 114

2,986

2,729

2 211

1 488

1,812 1,860 1,836 1,656

2,050

2 043

1,080

1,350

1 176

372

624

756

384

324

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 372 | 1 960 | 1,588 |
| February | 624 | 2,266 | 1 642 |
| March | 1,080 | 2,270 | 1 190 |
| April | 1 488 | 2,000 | 512 |
| May | 1 812 | 3 190 | 1 378 |
| June | 1 860 | 716 | - |
| July | 1 836 | 395 | - |
| August | 1 656 | 384 | - |
| September | 1 176 | 690 | - |
| October | 756 | 726 | - |
| November | 384 | 1 022 | 638 |
| December | 324 | 3 127 | 2,803 |
| TOTAL: | 13,368 | 18,746 | 9,751 |

| CAPEX (SPP cost) | 335,954.31 | UAH |
| --- | --- | --- |
| Power of SPP | 12 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 13,368 | kW/h |
| Consumption electricity an object | 18,746 | kW/h |
| Online purchase | 9,751 | kW/h |
| Coverage of consumption by own SPP | 8,995 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 4,373 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 58,506 | UAH |
| Costs on service by 1 year | 7,000 | UAH |
| The cost of projected energy consumption from own SPP | 53,970 | UAH |
| Cost consumption electricity without installed SPP | 112,476 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 46,970 | UAH |
| Payback of the project | 7.2 | years |
| The cost of generated electricity after payback | 0.52 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 12 KW.

No16

Location of SPP: Krupetsk territorial community

Krupetsk Lyceum

Description: Rooftop SPP

Power of the solar field:15 kW

Inverter power:12 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 27 | 8,282.90 ₴ | 223,638.30 ₴ |
| 2. Electricity inverter | Solis S5-GR3P12K | 1 | 54,975.00 ₴ | 54,975.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 14,843.25 ₴ | 14,843.25 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 120 | 40.00 ₴ | 4,800.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,800.00 ₴ | 3,800.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 6. Consumables |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 7. Transportation |  | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| 8. Assembly works |  | 1 | 42,000.00 ₴ | 42,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 394,356.00 ₴ |

CALCULATION

CAPEX

SPP 15 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 15 KW.

6,800

6 120

5,440

4,760

4,080

3,400

2,720

2,040

1 360

680

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

2,212 2,274 2,236

1 828

1 928

1 313

1 391

761

913

453	434

340

2,077	3986	5271	5197	2677	2824	3326	2230	596	976	2120	6762

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 453 | 2,077 | 1,624 |
| February | 761 | 3,986 | 3,225 |
| March | 1 313 | 5,271 | 3,958 |
| April | 1 828 | 5 197 | 3,369 |
| May | 2,212 | 2,677 | 465 |
| June | 2,274 | 2,824 | 550 |
| July | 2 236 | 3 326 | 1,090 |
| August | 1 928 | 2,230 | 302 |
| September | 1 391 | 596 | - |
| October | 913 | 976 | 63 |
| November | 434 | 2 120 | 1 686 |
| December | 340 | 6,762 | 6,422 |
| TOTAL: | 16,083 | 38,042 | 22,754 |

| CAPEX (SPP cost) | 394,356.00 | UAH |
| --- | --- | --- |
| Power of SPP | 15 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 16,083 | kW/h |
| Consumption electricity an object | 38,042 | kW/h |
| Online purchase | 22,754 | kW/h |
| Coverage of consumption by own SPP | 15,288 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 795 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 136,524 | UAH |
| Costs on service by 1 year | 8,000 | UAH |
| The cost of projected energy consumption from own SPP | 91,728 | UAH |
| Cost consumption electricity without installed SPP | 228 252 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 83,728 | UAH |
| Payback of the project | 4.7 | Years |
| The cost of generated electricity after payback | 0.50 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 15 KW.

No17

Location of SPP: Krupetsk territorial community

Polyan Gymnasium

Description: Rooftop SPP

Power of the solar field:8 kW

Inverter power:6 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 15 | 8,282.90 ₴ | 120,478.55 ₴ |
| 2. Electricity inverter | Solis S5-GR3P6K | 1 | 45,812.50 ₴ | 45,812.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 7,996.36 ₴ | 7,996.36 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 80 | 40.00 ₴ | 3,200.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,000.00 ₴ | 2,000.00 ₴ |
| 6. Consumables |  | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 7. Transportation |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 8. Assembly works |  | 1 | 22,400.00 ₴ | 22,400.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 249,186.86 ₴ |

CALCULATION

CAPEX

SPP 8 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 8 KW.

1 770

1 609

1 448

1 287

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

|  | 1 126 |
| --- | --- |
|  | 965 |
|  | 804 |
|  | 643 |
|  | 483 |
|  | 322 |
|  | 161 |
|  | 0 |

1 2081 240 1 224

1 104

992

720

784

504

416

248

256

216

1,800 1,393 1,591 1,314 1,158	555	727	1000	300	300	290	1789

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 248 | 1800 | 1 552 |
| February | 416 | 1393 | 977 |
| March | 720 | 1591 | 871 |
| April | 992 | 1314 | 322 |
| May | 1 208 | 1158 | - |
| June | 1,240 | 555 | - |
| July | 1 224 | 727 | - |
| August | 1 104 | 1000 | - |
| September | 784 | 300 | - |
| October | 504 | 300 | - |
| November | 256 | 290 | 34 |
| December | 216 | 1789 | 1 573 |
| TOTAL: | 8,912 | 12217 | 5329 |

| CAPEX (SPP cost) | 249,186.86 | UAH |
| --- | --- | --- |
| Power of SPP | 8 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 8,912 | kW/h |
| Consumption electricity an object | 12,217 | kW/h |
| Online purchase | 5,329 | kW/h |
| Coverage of consumption by own SPP | 6,888 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 2 024 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 31,974 | UAH |
| Costs on service by 1 year | 5,000 | UAH |
| The cost of projected energy consumption from own SPP | 41,328 | UAH |
| Cost consumption electricity without installed SPP | 73,302 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 36,328 | UAH |
| Payback of the project | 6.9 | years |
| The cost of generated electricity after payback | 0.56 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 8 KW.

No18

Location of SPP: Krupetsk territorial community

Holovliv Affiliate of Krupetsk Lyceum

Description: Rooftop SPP

Power of the solar field:8 kW

Inverter power:6 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 15 | 8,282.90 ₴ | 120,478.55 ₴ |
| 2. Electricity inverter | Solis S5-GR3P6K | 1 | 45,812.50 ₴ | 45,812.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 7,996.36 ₴ | 7,996.36 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 80 | 40.00 ₴ | 3,200.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,000.00 ₴ | 2,000.00 ₴ |
| 6. Consumables |  | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 7. Transportation |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 8. Assembly works |  | 1 | 22,400.00 ₴ | 22,400.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 249,186.86 ₴ |

CALCULATION

CAPEX

SPP 8 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 8 KW.

1,400

1,260

1 120

980

840

700

560

420

280

140

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

1 2081 240 1 224

1 104

992

784

720

504

416

248

256	216

998	409	1 151 1 160 1 072	911	457	86	155	907	1 004 1 368

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 248 | 998 | 750 |
| February | 416 | 409 | - |
| March | 720 | 1151 | 431 |
| April | 992 | 1160 | 168 |
| May | 1 208 | 1072 | - |
| June | 1,240 | 911 | - |
| July | 1 224 | 457 | - |
| August | 1 104 | 86 | - |
| September | 784 | 155 | - |
| October | 504 | 907 | 403 |
| November | 256 | 1004 | 748 |
| December | 216 | 1368 | 1 152 |
| TOTAL: | 8,912 | 9678 | 3652 |

| CAPEX (SPP cost) | 249,186.86 | UAH |
| --- | --- | --- |
| Power of SPP | 8 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 8,912 | kW/h |
| Consumption electricity an object | 9,678 | kW/h |
| Online purchase | 3,652 | kW/h |
| Coverage of consumption by own SPP | 6,026 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 2,886 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 21,912 | UAH |
| Costs on service by 1 year | 5,000 | UAH |
| The cost of projected energy consumption from own SPP | 36 156 | UAH |
| Cost consumption electricity without installed SPP | 58,068 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 31 156 | UAH |
| Payback of the project | 8.0 | years |
| The cost of generated electricity after payback | 0.56 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 8 KW.

No.

19

Location of SPP: Letychiv territorial community

Letychiv Lyceum

Description: Rooftop SPP

Power of the solar field: 15 kW

Inverter power: 12 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 27 | 8,282.90 ₴ | 223,638.30 ₴ |
| 2. Electricity inverter | Solis S5-GR3P12K | 1 | 54,975.00 ₴ | 54,975.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 14,843.25 ₴ | 14,843.25 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 120 | 40.00 ₴ | 4,800.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,800.00 ₴ | 3,800.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 6. Consumables |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 7. Transportation |  | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| 8. Assembly works |  | 1 | 42,000.00 ₴ | 42,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 394,356.00 ₴ |

CALCULATION

CAPEX

SPP 15 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 15 KW.

15,600

14,040

12,480

10,920

9,360

7,800

6,240

4,680

3 120

1,560

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

15,419

13,270

12 182

9,388

9,800

8,742

7,951

7,500

8,000

5,295

2,212 2,274 2,236

1 313 1 828	1 928 1 391

453

761

913

434

Jan	Feb	Mar    April	May  June

650

July

600

Aug

340

Sept	 Oct	Nov

Dec

SPP generation, kW/h	 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 453 | 12 182 | 11,729 |
| February | 761 | 5,295 | 4,534 |
| March | 1 313 | 15,419 | 14 106 |
| April | 1 828 | 13,270 | 11,442 |
| May | 2,212 | 9,388 | 7 176 |
| June | 2,274 | 7,951 | 5,677 |
| July | 2 236 | 650 | - |
| August | 1 928 | 600 | - |
| September | 1 391 | 7,500 | 6 109 |
| October | 913 | 8,000 | 7,087 |
| November | 434 | 8,742 | 8 308 |
| December | 340 | 9,800 | 9,460 |
| TOTAL: | 16,083 | 98,797 | 85,628 |

| CAPEX (SPP cost) | 394,356.00 | UAH |
| --- | --- | --- |
| Power of SPP | 15 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 16,083 | kW/h |
| Consumption electricity an object | 98,797 | kW/h |
| Online purchase | 85,628 | kW/h |
| Coverage of consumption by own SPP | 13 169 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 2,914 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 513,768 | UAH |
| Costs on service by 1 year | 8,000 | UAH |
| The cost of projected energy consumption from own SPP | 79,014 | UAH |
| Cost consumption electricity without installed SPP | 592,782 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 71,014 | UAH |
| Payback of the project | 5,6 | Years |
| The cost of generated electricity after payback | 0.50 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 15 KW.

No. 20

Location of SPP: Novodunayeveti territorial community

Treatment Plant

Description: Rooftop SPP

Power of the solar field: 15 kW

Inverter power: 12 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 27 | 8,282.90 ₴ | 223,638.30 ₴ |
| 2. Electricity inverter | Solis S5-GR3P12K | 1 | 54,975.00 ₴ | 54,975.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 14,843.25 ₴ | 14,843.25 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 120 | 40.00 ₴ | 4,800.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,800.00 ₴ | 3,800.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 6. Consumables |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 7. Transportation |  | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| 8. Assembly works |  | 1 | 42,000.00 ₴ | 42,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 394,356.00 ₴ |

CALCULATION

CAPEX

SPP 15 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 15 KW.

8,000

7,200

6,400

5,600

4,800

4,000

3,200

2,400

1,600

800

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

7,500	7500	7500	7500	7500	7500	7500	7500	7500	7500	7500	7500

2,212 2,274 2,236

1 828

1 928

1 313

1 391

913

453

761

434

340

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 453 | 7,500 | 7,047 |
| February | 761 | 7,500 | 6,739 |
| March | 1 313 | 7,500 | 6 187 |
| April | 1 828 | 7,500 | 5,672 |
| May | 2,212 | 7,500 | 5,288 |
| June | 2,274 | 7,500 | 5,226 |
| July | 2 236 | 7,500 | 5,264 |
| August | 1 928 | 7,500 | 5,572 |
| September | 1 391 | 7,500 | 6 109 |
| October | 913 | 7,500 | 6,587 |
| November | 434 | 7,500 | 7,066 |
| December | 340 | 7,500 | 7 160 |
| TOTAL: | 16,083 | 90,000 | 73,917 |

| CAPEX (SPP cost) | 394,356.00 | UAH |
| --- | --- | --- |
| Power of SPP | 15 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 16,083 | kW/h |
| Consumption electricity an object | 90,000 | kW/h |
| Online purchase | 73,917 | kW/h |
| Coverage of consumption by own SPP | 16,083 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 0 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 443 502 | UAH |
| Costs on service by 1 year | 8,000 | UAH |
| The cost of projected energy consumption from own SPP | 96 498 | UAH |
| Cost consumption electricity without installed SPP | 540,000 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 88,498 | UAH |
| Payback of the project | 4.5 | Years |
| The cost of generated electricity after payback | 0.50 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 15 KW.

No21

Location of SPP: Novoushytsk settlement territorial community

SE "Miskvodokanal"

Description: Rooftop SPP

Power of the solar field: 30 kW

Inverter power: 30 kW

CALCULATION

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 55 | 8,466.15 ₴ | 461,790.00 ₴ |
| 2. Electricity inverter | SolisS5-GC30K | 1 | 76,965.00 ₴ | 76,965.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 39,981.82 ₴ | 39,981.82 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X25 | 10 | 800.00 ₴ | 800.00 ₴ |
| cable for solar panels | KBE | 200 | 40.00 ₴ | 8,000.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 5,500.00 ₴ | 5,500.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 3,500.00 ₴ | 3,500.00 ₴ |
| 6. Consumables |  | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| 7. Transportation |  | 1 | 10,000.00 ₴ | 10,000.00 ₴ |
| 8. Assembly works |  | 1 | 84,000.00 ₴ | 84,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 734,736.27 ₴ |

CAPEX

SPP 30 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 30 KW.

12,200

10,980

9,760

8,540

7,320

6 100

4,880

3,660

2 440

1,220

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

12,080

11,720

11,160

9,760

9,320

9,680	9,720

9,280

8,720

8,300

8,000	8000

4,5304,650 4,590

3,720

4 140

2,700

2,940

1,560	1 890

930	960

810

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 930 | 9,760 | 8,830 |
| February | 1,560 | 8,720 | 7 160 |
| March | 2,700 | 9,320 | 6,620 |
| April | 3,720 | 9,680 | 5,960 |
| May | 4,530 | 9,720 | 5 190 |
| June | 4,650 | 9,280 | 4,630 |
| July | 4,590 | 12,080 | 7,490 |
| August | 4 140 | 11,160 | 7,020 |
| September | 2,940 | 11,720 | 8,780 |
| October | 1 890 | 8,300 | 6,410 |
| November | 960 | 8,000 | 7,040 |
| December | 810 | 8,000 | 7 190 |
| TOTAL: | 33,420 | 115,740 | 82,320 |

| CAPEX (SPP cost) | 734,736.27 | UAH |
| --- | --- | --- |
| Power of SPP | 30 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 33,420 | kW/h |
| Consumption electricity an object | 115,740 | kW/h |
| Online purchase | 82,320 | kW/h |
| Coverage of consumption by own SPP | 33,420 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 0 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 493,920 | UAH |
| Costs on service by 1 year | 17,000 | UAH |
| The cost of projected energy consumption from own SPP | 200,520 | UAH |
| Cost consumption electricity without installed SPP | 694 440 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 183,520 | UAH |
| Payback of the project | 4.0 | Years |
| The cost of generated electricity after payback | 0.51 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 30 KW.

No. 22

Location of SPP : Smotrych territorial community

Smotrych Lyceum

Description: Rooftop SPP

Power of the solar field: 12 kW

Inverter power: 10 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 22 | 8,282.90 ₴ | 180,717.82 ₴ |
| 2. Electricity inverter | Solis S5-GR3P10K | 1 | 53,142.50 ₴ | 53,142.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 11,994.55 ₴ | 11,994.55 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 100 | 40.00 ₴ | 4,000.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,500.00 ₴ | 3,500.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,600.00 ₴ | 3,600.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,300.00 ₴ | 2,300.00 ₴ |
| 6. Consumables |  | 1 | 2,800.00 ₴ | 2,800.00 ₴ |
| 7. Transportation |  | 1 | 4,500.00 ₴ | 4,500.00 ₴ |
| 8. Assembly works |  | 1 | 33,600.00 ₴ | 33,600.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 335,954.31 ₴ |

CALCULATION

CAPEX

SPP 12 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 12 KW.

7,000

6,300

5,600

4,900

4,200

3,500

2,800

2 100

1,400

700

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

1 488

1,812 1,860 1,836 1,656

1,080

1 176

372

624

756

384

6,200 6,500 5,200 2,700 2,500	860	890

860	1 700 3,500 4,500 7,000

324

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 372 | 6,200 | 5,828 |
| February | 624 | 6,500 | 5,876 |
| March | 1,080 | 5,200 | 4 120 |
| April | 1 488 | 2,700 | 1 212 |
| May | 1 812 | 2,500 | 688 |
| June | 1 860 | 860 | - |
| July | 1 836 | 890 | - |
| August | 1 656 | 860 | - |
| September | 1 176 | 1,700 | 524 |
| October | 756 | 3,500 | 2,744 |
| November | 384 | 4,500 | 4 116 |
| December | 324 | 7,000 | 6,676 |
| TOTAL: | 13,368 | 42,410 | 31,784 |

| CAPEX (SPP cost) | 335,954.31 | UAH |
| --- | --- | --- |
| Power of SPP | 12 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 13,368 | kW/h |
| Consumption electricity an object | 42,410 | kW/h |
| Online purchase | 31,784 | kW/h |
| Coverage of consumption by own SPP | 10,626 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 2,742 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 190,704 | UAH |
| Costs on service by 1 year | 7,000 | UAH |
| The cost of projected energy consumption from own SPP | 63,756 | UAH |
| Cost consumption electricity without installed SPP | 254,460 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 56,756 | UAH |
| Payback of the project | 5.9 | years |
| The cost of generated electricity after payback | 0.52 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 12 KW.

No23

Location of SPP: Khmelnytskyi territorial community

Khmelnytskyi City Children's Hospital

Description: Rooftop SPP

Power of the solar field: 30 kW

Inverter power: 30 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 55 | 8,466.15 ₴ | 461,790.00 ₴ |
| 2. Electricity inverter | SolisS5-GC30K | 1 | 76,965.00 ₴ | 76,965.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 39,981.82 ₴ | 39,981.82 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X25 | 10 | 800.00 ₴ | 800.00 ₴ |
| cable for solar panels | KBE | 200 | 40.00 ₴ | 8,000.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 5,500.00 ₴ | 5,500.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 3,500.00 ₴ | 3,500.00 ₴ |
| 6. Consumables |  | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| 7. Transportation |  | 1 | 10,000.00 ₴ | 10,000.00 ₴ |
| 8. Assembly works |  | 1 | 84,000.00 ₴ | 84,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 734,736.27 ₴ |

CALCULATION

CAPEX

SPP 30 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 30 KW.

34,381

33 133

32,986

28,668

29,613 28,788

26,512

27,026

24,073

24,790

23,514

22,032

2,700

3,7204 530 4 650 4 590 4 140 2 940

930

January

1,560

February

1 890	960

Mar

Apr

May

Jun

July

Aug

Sept	 Oct	Nov

810

Dec

34,400

30,960

27,520

24,080

20,640

17,200

13,760

10,320

6,880

3 440

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

SPP generation, kW/h	 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 930 | 33 133 | 32,203 |
| February | 1,560 | 28,668 | 27 108 |
| March | 2,700 | 24,073 | 21,373 |
| April | 3,720 | 26,512 | 22,792 |
| May | 4,530 | 22,032 | 17,502 |
| June | 4,650 | 23,514 | 18,864 |
| July | 4,590 | 24,790 | 20,200 |
| August | 4 140 | 27,026 | 22,886 |
| September | 2,940 | 32,986 | 30,046 |
| October | 1 890 | 34,381 | 32,491 |
| November | 960 | 29,613 | 28,653 |
| December | 810 | 28,788 | 27,978 |
| TOTAL: | 33,420 | 335,516 | 302,096 |

| CAPEX (SPP cost) | 734,736.27 | UAH |
| --- | --- | --- |
| Power of SPP | 30 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 33,420 | kW/h |
| Consumption electricity an object | 335,516 | kW/h |
| Online purchase | 302,096 | kW/h |
| Coverage of consumption by own SPP | 33,420 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 0 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 1,812,576 | UAH |
| Costs on service by 1 year | 17,000 | UAH |
| The cost of projected energy consumption from own SPP | 200,520 | UAH |
| Cost consumption electricity without installed SPP | 2,013,096 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 183,520 | UAH |
| Payback of the project | 4.0 | Years |
| The cost of generated electricity after payback | 0.51 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 30 KW.

No. 24

Location of SPP: Khmelnytskyi territorial community

SE Khmelnytskyi City Hospital, Main building

Description: Rooftop SPP

Power of the solar field: 30 kW

Inverter power: 30 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 55 | 8,466.15 ₴ | 461,790.00 ₴ |
| 2. Electricity inverter | SolisS5-GC30K | 1 | 76,965.00 ₴ | 76,965.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 39,981.82 ₴ | 39,981.82 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X25 | 10 | 800.00 ₴ | 800.00 ₴ |
| cable for solar panels | KBE | 200 | 40.00 ₴ | 8,000.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 5,500.00 ₴ | 5,500.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 3,500.00 ₴ | 3,500.00 ₴ |
| 6. Consumables |  | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| 7. Transportation |  | 1 | 10,000.00 ₴ | 10,000.00 ₴ |
| 8. Assembly works |  | 1 | 84,000.00 ₴ | 84,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 734,736.27 ₴ |

CALCULATION

CAPEX

SPP 30 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 30 KW.

36,353

32,833

32,332

33,517

32,396

31 471

29,554

29,304

26,677 25,904 25,546

24,627

2,700

3,7204 530 4 650 4 590 4 140 2 940

930

January

1,560

February

1 890

Mar

Apr

May

Jun

July

Aug

Sept	 Oct

960	810

Nov	Dec

36,600

32,940

29,280

25,620

21,960

18,300

14,640

10,980

7,320

3,660

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

SPP generation, kW/h	 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 930 | 32,833 | 31,903 |
| February | 1,560 | 29,554 | 27,994 |
| March | 2,700 | 32,332 | 29,632 |
| April | 3,720 | 29,304 | 25,584 |
| May | 4,530 | 26,677 | 22 147 |
| June | 4,650 | 25,904 | 21,254 |
| July | 4,590 | 25,546 | 20,956 |
| August | 4 140 | 24,627 | 20,487 |
| September | 2,940 | 32,396 | 29,456 |
| October | 1 890 | 33,517 | 31,627 |
| November | 960 | 31 471 | 30,511 |
| December | 810 | 36,353 | 35,543 |
| TOTAL: | 33,420 | 360,514 | 327,094 |

| CAPEX (SPP cost) | 734,736.27 | UAH |
| --- | --- | --- |
| Power of SPP | 30 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 33,420 | kW/h |
| Consumption electricity an object | 360,514 | kW/h |
| Online purchase | 327,094 | kW/h |
| Coverage of consumption by own SPP | 33,420 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 0 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 1,962,564 | UAH |
| Costs on service by 1 year | 17,000 | UAH |
| The cost of projected energy consumption from own SPP | 200,520 | UAH |
| Cost consumption electricity without installed SPP | 2,163,084 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 183,520 | UAH |
| Payback of the project | 4.0 | Years |
| The cost of generated electricity after payback | 0.51 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 30 KW.

No. 25

Location of SPP: Khmelnytskyi territorial community

Economic building of Maternity Hospital

Description: Rooftop SPP

Power of the solar field: 15 kW

Inverter power: 12 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 27 | 8,282.90 ₴ | 223,638.30 ₴ |
| 2. Electricity inverter | Solis S5-GR3P12K | 1 | 54,975.00 ₴ | 54,975.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 14,843.25 ₴ | 14,843.25 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 120 | 40.00 ₴ | 4,800.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,800.00 ₴ | 3,800.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 6. Consumables |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 7. Transportation |  | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| 8. Assembly works |  | 1 | 42,000.00 ₴ | 42,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 394,356.00 ₴ |

CALCULATION

CAPEX

SPP 15 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 15 KW.

4,800

4,320

3,840

3,360

2,880

2,400

1 920

1 440

960

480

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

2,2122,274 2,236

1 828

1 928

1 313

1 391

761

913

453

434

340

4,377	4056	4518	3735	2840	2770	2108	2933	3270	4209	4383	4625

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 453 | 4,377 | 3,924 |
| February | 761 | 4,056 | 3,295 |
| March | 1 313 | 4,518 | 3 205 |
| April | 1 828 | 3,735 | 1 907 |
| May | 2,212 | 2,840 | 628 |
| June | 2,274 | 2,770 | 496 |
| July | 2 236 | 2 108 | - |
| August | 1 928 | 2,933 | 1 005 |
| September | 1 391 | 3,270 | 1 879 |
| October | 913 | 4 209 | 3,296 |
| November | 434 | 4,383 | 3,949 |
| December | 340 | 4,625 | 4,285 |
| TOTAL: | 16,083 | 43,824 | 27,869 |

| CAPEX (SPP cost) | 394,356.00 | UAH |
| --- | --- | --- |
| Power of SPP | 15 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 16,083 | kW/h |
| Consumption electricity an object | 43,824 | kW/h |
| Online purchase | 27,869 | kW/h |
| Coverage of consumption by own SPP | 15,955 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 128 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 167,214 | UAH |
| Costs on service by 1 year | 8,000 | UAH |
| The cost of projected energy consumption from own SPP | 95,730 | UAH |
| Cost consumption electricity without installed SPP | 262,944 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 87,730 | UAH |
| Payback of the project | 4.5 | Years |
| The cost of generated electricity after payback | 0.50 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 15 KW.

No26

Location of SPP: Khmelnytskyi territorial community

Khmelnytskyi City Hospital

Description: Rooftop SPP

Power of the solar field: 30 kW

Inverter power: 30 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 55 | 8,466.15 ₴ | 461,790.00 ₴ |
| 2. Electricity inverter | SolisS5-GC30K | 1 | 76,965.00 ₴ | 76,965.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 39,981.82 ₴ | 39,981.82 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X25 | 10 | 800.00 ₴ | 800.00 ₴ |
| cable for solar panels | KBE | 200 | 40.00 ₴ | 8,000.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 5,500.00 ₴ | 5,500.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 3,500.00 ₴ | 3,500.00 ₴ |
| 6. Consumables |  | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| 7. Transportation |  | 1 | 10,000.00 ₴ | 10,000.00 ₴ |
| 8. Assembly works |  | 1 | 84,000.00 ₴ | 84,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 734,736.27 ₴ |

CALCULATION

CAPEX

SPP 30 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 30 KW.

107 433

99,588

92,830

87,720

78 457

74,924

65,90567,649

65 905 63 989

51 003

45,504

930

January

1,560 2,700 3,7204,530 4,650 4,590 4,140 2,940 1,890

960

February

Mar

Apr

May

June

July

Aug

Sept	Oct	Nov

810

Dec

108,000

97,200

86,400

75,600

64,800

54,000

43,200

32,400

21,600

10,800

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

SPP generation, kW/h	 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 930 | 87,720 | 86,790 |
| February | 1,560 | 78 457 | 76,897 |
| March | 2,700 | 65,905 | 63,205 |
| April | 3,720 | 67,649 | 63,929 |
| May | 4,530 | 74,924 | 70,394 |
| June | 4,650 | 65,905 | 61,255 |
| July | 4,590 | 63,989 | 59,399 |
| August | 4 140 | 51 003 | 46,863 |
| September | 2,940 | 45,504 | 42,564 |
| October | 1 890 | 99,588 | 97,698 |
| November | 960 | 107 433 | 106,473 |
| December | 810 | 92,830 | 92,020 |
| TOTAL: | 33,420 | 900 907 | 867 487 |

| CAPEX (SPP cost) | 734,736.27 | UAH |
| --- | --- | --- |
| Power of SPP | 30 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 33,420 | kW/h |
| Consumption electricity an object | 900 907 | kW/h |
| Online purchase | 867 487 | kW/h |
| Coverage of consumption by own SPP | 33,420 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 0 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 5,204,922 | UAH |
| Costs on service by 1 year | 17,000 | UAH |
| The cost of projected energy consumption from own SPP | 200,520 | UAH |
| Cost consumption electricity without installed SPP | 5,405,442 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 183,520 | UAH |
| Payback of the project | 4.0 | Years |
| The cost of generated electricity after payback | 0.51 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 30 KW.

No. 27

Location of SPP: Khmelnytsyi territorial community

Khmelnytsky Infectious Diseases Hospital

Description: Rooftop SPP

Power of the solar field: 30 kW

Inverter power: 30 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 55 | 8,466.15 ₴ | 461,790.00 ₴ |
| 2. Electricity inverter | SolisS5-GC30K | 1 | 76,965.00 ₴ | 76,965.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 39,981.82 ₴ | 39,981.82 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X25 | 10 | 800.00 ₴ | 800.00 ₴ |
| cable for solar panels | KBE | 200 | 40.00 ₴ | 8,000.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 5,500.00 ₴ | 5,500.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 3,500.00 ₴ | 3,500.00 ₴ |
| 6. Consumables |  | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| 7. Transportation |  | 1 | 10,000.00 ₴ | 10,000.00 ₴ |
| 8. Assembly works |  | 1 | 84,000.00 ₴ | 84,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 734,736.27 ₴ |

CALCULATION

CAPEX

SPP 30 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 30 KW.

71,400

64,260

57 120

49,980

42,840

35,700

28,560

21,420

14,280

7 140

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

71 148

61 14161,938

49,396

44,746

42 44142,801

36,762

38,067

28,677

24,310

17,660

930

January

1,560

February

2,700 3,7204 530 4 650 4 590 4 140 2 940

1 890

960

Mar	Apr	May	June	July	Aug	Sept	Oct	Nov

810

Dec

SPP generation, kW/h	 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 930 | 44,746 | 43,816 |
| February | 1,560 | 36,762 | 35,202 |
| March | 2,700 | 42 441 | 39,741 |
| April | 3,720 | 42,801 | 39,081 |
| May | 4,530 | 49,396 | 44,866 |
| June | 4,650 | 28,677 | 24,027 |
| July | 4,590 | 24,310 | 19,720 |
| August | 4 140 | 17,660 | 13,520 |
| September | 2,940 | 38,067 | 35 127 |
| October | 1 890 | 61 141 | 59,251 |
| November | 960 | 61,938 | 60,978 |
| December | 810 | 71 148 | 70,338 |
| TOTAL: | 33,420 | 519,087 | 485,667 |

| CAPEX (SPP cost) | 734,736.27 | UAH |
| --- | --- | --- |
| Power of SPP | 30 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 33,420 | kW/h |
| Consumption electricity an object | 519,087 | kW/h |
| Online purchase | 485,667 | kW/h |
| Coverage of consumption by own SPP | 33,420 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 0 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 2,914,002 | UAH |
| Costs on service by 1 year | 17,000 | UAH |
| The cost of projected energy consumption from own SPP | 200,520 | UAH |
| Cost consumption electricity without installed SPP | 3,114,522 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 183,520 | UAH |
| Payback of the project | 4.0 | Years |
| The cost of generated electricity after payback | 0.51 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 30 KW.

No. 28

Location of SPP : Shchyborivka territorial community

Kuzmin Lyceum

Description: Rooftop SPP

Power of the solar field: 12 kW

Inverter power: 10 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 22 | 8,282.90 ₴ | 180,717.82 ₴ |
| 2. Electricity inverter | Solis S5-GR3P10K | 1 | 53,142.50 ₴ | 53,142.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 11,994.55 ₴ | 11,994.55 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 100 | 40.00 ₴ | 4,000.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,500.00 ₴ | 3,500.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,600.00 ₴ | 3,600.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,300.00 ₴ | 2,300.00 ₴ |
| 6. Consumables |  | 1 | 2,800.00 ₴ | 2,800.00 ₴ |
| 7. Transportation |  | 1 | 4,500.00 ₴ | 4,500.00 ₴ |
| 8. Assembly works |  | 1 | 33,600.00 ₴ | 33,600.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 335,954.31 ₴ |

CALCULATION

CAPEX

SPP 12 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 12 KW.

11,000

9,900

8,800

7,700

6,600

5,500

4,400

3,300

2,200

1 100

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

1 488

1 812	1 860	1 836

1 656

1,080

1 176

10,804 7,170

372

624

0	70	1589	78	169	2780	1250

Jan	Feb	Mar	Apr	May	June	July	Aug	Sept

756

384

7,250	6300

Oct	Nov

6,507

Dec

324

SPP generation, kW/h	 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 372 | 10,804 | 10,432 |
| February | 624 | 7 170 | 6,546 |
| March | 1,080 | 0 | - |
| April | 1 488 | 70 | - |
| May | 1 812 | 1,589 | - |
| June | 1 860 | 78 | - |
| July | 1 836 | 169 | - |
| August | 1 656 | 2,780 | 1 124 |
| September | 1 176 | 1,250 | 74 |
| October | 756 | 7,250 | 6,494 |
| November | 384 | 6,300 | 5,916 |
| December | 324 | 6,507 | 6 183 |
| TOTAL: | 13,368 | 43,967 | 36769 |

| CAPEX (SPP cost) | 335,954.31 | UAH |
| --- | --- | --- |
| Power of SPP | 12 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 13,368 | kW/h |
| Consumption electricity an object | 43,967 | kW/h |
| Online purchase | 36,769 | kW/h |
| Coverage of consumption by own SPP | 7 198 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 6 170 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 220,614 | UAH |
| Costs on service by 1 year | 7,000 | UAH |
| The cost of projected energy consumption from own SPP | 43 188 | UAH |
| Cost consumption electricity without installed SPP | 263,802 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 36 188 | UAH |
| Payback of the project | 9.3 | years |
| The cost of generated electricity after payback | 0.52 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 12 KW.

No29

Location of SPP: Shchyborivka territorial community

Secondary school, preschool, village club of vil. Radisne

Description: Rooftop SPP

Power of the solar field:15 kW

Inverter power:12 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 27 | 8,282.90 ₴ | 223,638.30 ₴ |
| 2. Electricity inverter | Solis S5-GR3P12K | 1 | 54,975.00 ₴ | 54,975.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 14,843.25 ₴ | 14,843.25 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 120 | 40.00 ₴ | 4,800.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,800.00 ₴ | 3,800.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 6. Consumables |  | 1 | 3,000.00 ₴ | 3,000.00 ₴ |
| 7. Transportation |  | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| 8. Assembly works |  | 1 | 42,000.00 ₴ | 42,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 394,356.00 ₴ |

CALCULATION

CAPEX

SPP 15 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 15 KW.

3,400

3,060

2,720

2,380

2,040

1,700

1 360

1,020

680

340

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

2,2122,274 2,236

1 828	1 928

1 313

1 391

913

761

453

434	340

2 551 3 140	554	1879	850	93	406	1 786 1,800 2,403 3,357 1,693

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 453 | 2 551 | 2,098 |
| February | 761 | 3 140 | 2 379 |
| March | 1 313 | 554 | - |
| April | 1 828 | 1 879 | 51 |
| May | 2,212 | 850 | - |
| June | 2,274 | 93 | - |
| July | 2 236 | 406 | - |
| August | 1 928 | 1 786 | - |
| September | 1 391 | 1,800 | 409 |
| October | 913 | 2 403 | 1 490 |
| November | 434 | 3 357 | 2,923 |
| December | 340 | 1 693 | 1 353 |
| TOTAL: | 16,083 | 20,512 | 10,703 |

| CAPEX (SPP cost) | 394,356.00 | UAH |
| --- | --- | --- |
| Power of SPP | 15 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 16,083 | kW/h |
| Consumption electricity an object | 20,512 | kW/h |
| Online purchase | 10,703 | kW/h |
| Coverage of consumption by own SPP | 9,809 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 6,274 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 64,218 | UAH |
| Costs on service by 1 year | 8,000 | UAH |
| The cost of projected energy consumption from own SPP | 58,854 | UAH |
| Cost consumption electricity without installed SPP | 123,072 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 50,854 | UAH |
| Payback of the project | 7,8 | Years |
| The cost of generated electricity after payback | 0.50 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 15 KW.

No. 30

Location of SPP: Berezdiv territorial community

Berezdiv Lyceum

Description: Rooftop SPP

Power of the solar field: 12 kW

Inverter power: 10 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 22 | 8,282.90 ₴ | 180,717.82 ₴ |
| 2. Electricity inverter | Solis S5-GR3P10K | 1 | 53,142.50 ₴ | 53,142.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 11,994.55 ₴ | 11,994.55 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 100 | 40.00 ₴ | 4,000.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,500.00 ₴ | 3,500.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,600.00 ₴ | 3,600.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,300.00 ₴ | 2,300.00 ₴ |
| 6. Consumables |  | 1 | 2,800.00 ₴ | 2,800.00 ₴ |
| 7. Transportation |  | 1 | 4,500.00 ₴ | 4,500.00 ₴ |
| 8. Assembly works |  | 1 | 33,600.00 ₴ | 33,600.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 335,954.31 ₴ |

CALCULATION

CAPEX

SPP 12 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 12 KW.

6,900

6,210

5,520

4,830

4 140

3,450

2,760

2,070

1,380

690

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

1 488

1,812 1,860 1,836 1,656

1,080

1 176

372

624

756

384

324

3,820 5,012 6,575 2,010	595	267	88

90	1 140 2 085 5 927 6 884

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 372 | 3,820 | 3 448 |
| February | 624 | 5,012 | 4,388 |
| March | 1,080 | 6,575 | 5,495 |
| April | 1 488 | 2010 | 522 |
| May | 1 812 | 595 | - |
| June | 1 860 | 267 | - |
| July | 1 836 | 88 | - |
| August | 1 656 | 90 | - |
| September | 1 176 | 1 140 | - |
| October | 756 | 2,085 | 1 329 |
| November | 384 | 5,927 | 5,543 |
| December | 324 | 6,884 | 6,560 |
| TOTAL: | 13,368 | 34,493 | 27285 |

| CAPEX (SPP cost) | 335,954.31 | UAH |
| --- | --- | --- |
| Power of SPP | 12 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 13,368 | kW/h |
| Consumption electricity an object | 34,493 | kW/h |
| Online purchase | 27,285 | kW/h |
| Coverage of consumption by own SPP | 7,208 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 6 160 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 163,710 | UAH |
| Costs on service by 1 year | 7,000 | UAH |
| The cost of projected energy consumption from own SPP | 43,248 | UAH |
| Cost consumption electricity without installed SPP | 206,958 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 36,248 | UAH |
| Payback of the project | 9.3 | years |
| The cost of generated electricity after payback | 0.52 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 12 KW.

No31

Location of SPP: Dunayevtsi territorial community. SE "Miskwodokanal" Treatment plant

Description: Rooftop SPP

Power of the solar field: 30 kW

Inverter power: 30 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 55 | 8,466.15 ₴ | 461,790.00 ₴ |
| 2. Electricity inverter | SolisS5-GC30K | 1 | 76,965.00 ₴ | 76,965.00 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 39,981.82 ₴ | 39,981.82 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X25 | 10 | 800.00 ₴ | 800.00 ₴ |
| cable for solar panels | KBE | 200 | 40.00 ₴ | 8,000.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 5,000.00 ₴ | 5,000.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 5,500.00 ₴ | 5,500.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 3,500.00 ₴ | 3,500.00 ₴ |
| 6. Consumables |  | 1 | 4,000.00 ₴ | 4,000.00 ₴ |
| 7. Transportation |  | 1 | 10,000.00 ₴ | 10,000.00 ₴ |
| 8. Assembly works |  | 1 | 84,000.00 ₴ | 84,000.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 734,736.27 ₴ |

CALCULATION

CAPEX

SPP 30 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 30 KW.

30,000 30,000 30,000 30,000 30,000 30,000 30,000 30,000 30,000 30,000 30,000 30,000

2,700

3,7204,530 4,650 4,590 4,140

2,940

930

January

1,560

February

1 890	960

Mar

Apr

May

June

July

Aug

Sept	Oct	Nov

810

Dec

32,000

28,800

25,600

22,400

19,200

16,000

12,800

9,600

6,400

3,200

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

SPP generation, kW/h	 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 930 | 30,000 | 29,070 |
| February | 1,560 | 30,000 | 28,440 |
| March | 2,700 | 30,000 | 27,300 |
| April | 3,720 | 30,000 | 26,280 |
| May | 4,530 | 30,000 | 25,470 |
| June | 4,650 | 30,000 | 25,350 |
| July | 4,590 | 30,000 | 25,410 |
| August | 4 140 | 30,000 | 25,860 |
| September | 2,940 | 30,000 | 27,060 |
| October | 1 890 | 30,000 | 28 110 |
| November | 960 | 30,000 | 29,040 |
| December | 810 | 30,000 | 29,190 |
| TOTAL: | 33,420 | 360,000 | 326,580 |

| CAPEX (SPP cost) | 734,736.27 | UAH |
| --- | --- | --- |
| Power of SPP | 30 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 33,420 | kW/h |
| Consumption electricity an object | 360,000 | kW/h |
| Online purchase | 326,580 | kW/h |
| Coverage of consumption by own SPP | 33,420 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 0 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 1,959,480 | UAH |
| Costs on service by 1 year | 17,000 | UAH |
| The cost of projected energy consumption from own SPP | 200,520 | UAH |
| Cost consumption electricity without installed SPP | 2,160,000 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 183,520 | UAH |
| Payback of the project | 4.0 | Years |
| The cost of generated electricity after payback | 0.51 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 30 KW.

No. 32

Location of SPP: City SE “Misksvitlo” Admin building

Description: Rooftop SPP

Power of the solar field: 12 kW

Inverter power: 10 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 22 | 8,282.90 ₴ | 180,717.82 ₴ |
| 2. Electricity inverter | Solis S5-GR3P10K | 1 | 53,142.50 ₴ | 53,142.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 11,994.55 ₴ | 11,994.55 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 100 | 40.00 ₴ | 4,000.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 3,500.00 ₴ | 3,500.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 3,600.00 ₴ | 3,600.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 2,300.00 ₴ | 2,300.00 ₴ |
| 6. Consumables |  | 1 | 2,800.00 ₴ | 2,800.00 ₴ |
| 7. Transportation |  | 1 | 4,500.00 ₴ | 4,500.00 ₴ |
| 8. Assembly works |  | 1 | 33,600.00 ₴ | 33,600.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 335,954.31 ₴ |

CALCULATION

CAPEX

SPP 12 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 12 KW.

3,400

3,060

2,720

2,380

2,040

1,700

1 360

1,020

680

340

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

1 812

1 860

1 836

1 656

1 488

1,080

1 176

756

624

372

384

324

3,200	3000	2800	2500	2500	2000	2000	2000	2500	2800	3000	3200

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 372 | 3,200 | 2,828 |
| February | 624 | 3,000 | 2 376 |
| March | 1,080 | 2,800 | 1,720 |
| April | 1 488 | 2,500 | 1 012 |
| May | 1 812 | 2,500 | 688 |
| June | 1 860 | 2,000 | 140 |
| July | 1 836 | 2,000 | 164 |
| August | 1 656 | 2,000 | 344 |
| September | 1 176 | 2,500 | 1 324 |
| October | 756 | 2,800 | 2 044 |
| November | 384 | 3,000 | 2,616 |
| December | 324 | 3,200 | 2,876 |
| TOTAL: | 13,368 | 31,500 | 18 132 |

| CAPEX (SPP cost) | 335,954.31 | UAH |
| --- | --- | --- |
| Power of SPP | 12 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 13,368 | kW/h |
| Consumption electricity an object | 31,500 | kW/h |
| Online purchase | 18 132 | kW/h |
| Coverage of consumption by own SPP | 13,368 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 0 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 108,792 | UAH |
| Costs on service by 1 year | 7,000 | UAH |
| The cost of projected energy consumption from own SPP | 80,208 | UAH |
| Cost consumption electricity without installed SPP | 189,000 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 73 208 | UAH |
| Payback of the project | 4.6 | years |
| The cost of generated electricity after payback | 0.52 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 12 KW.

No33

Location of SPP: City SE “Misksvitlo” Workshop

Description: Rooftop SPP

Power of the solar field: 3.3 kW

Inverter power: 5 kW

| Name | Description | K- St | Costunits | Price |
| --- | --- | --- | --- | --- |
| 1. Solar panel | Risen 550W | 6 | 8,282.90 ₴ | 49,697.40 ₴ |
| 2. Electricity inverter | Solis S5-GR3P10K | 1 | 53,142.50 ₴ | 53,142.50 ₴ |
| 3. The controller for the export of electricity to the network | Solis EPM3-5G plus | 1 | 23,199.45 ₴ | 23,199.45 ₴ |
| 4. Assembly structure |  |  |  |  |
| Type of metal construction | aluminum | 1 | 3,298.50 ₴ | 3,298.50 ₴ |
| 5. Electrical products |  |  |  |  |
| cable | AVVG4X16 | 10 | 600.00 ₴ | 600.00 ₴ |
| cable for solar panels | KBE | 20 | 40.00 ₴ | 800.00 ₴ |
| automatics protection DC | ETI fuSPP, shield ETI | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| automatics protection AC | shield ETI, automatichull Hager,Shnider SSRIs | 1 | 2,600.00 ₴ | 2,600.00 ₴ |
| switching facilities | MC4 connectors, tips, copper wire, grounding bus | 1 | 1,800.00 ₴ | 1,800.00 ₴ |
| 6. Consumables |  | 1 | 1,500.00 ₴ | 1,500.00 ₴ |
| 7. Transportation |  | 1 | 2,500.00 ₴ | 2,500.00 ₴ |
| 8. Assembly works |  | 1 | 9,240.00 ₴ | 9,240.00 ₴ |
| 9. Design and estimate documentation |  | 1 | 12,000.00 ₴ | 12,000.00 ₴ |
| CAPEX |  |  | Total: | 162,877.85 ₴ |

CALCULATION

CAPEX

SPP 3.3 KW.

INDICATORS GENERATIONS AND CONSUMPTION

SPP 3.3 KW.

550

495

440

385

330

275

220

165

110

55

0

SCHEDULE OF SPP GENERATION AND ELECTRICITY CONSUMPTION BY THE FACILITY

498

511

504

455

409

323

297

207

171

102

105	89

300	300	300	300	500	400	400	400	500	300	300	300

Jan	Feb	Mar	April	May	June	July	Aug	Sept	Oct	Nov   Dec SPP generation, kW/h			 Electricity consumption, kW/h

|  | SPP generation, kW/h | Consumption object, kW/h | Purchase from the network, kWh |
| --- | --- | --- | --- |
| January | 102.3 | 300 | 197.7 |
| February | 171.6 | 300 | 128.4 |
| March | 297 | 300 | 3 |
| April | 409.2 | 300 | - |
| May | 498.3 | 500 | 1.7 |
| June | 511.5 | 400 | - |
| July | 504.9 | 400 | - |
| August | 455.4 | 400 | - |
| September | 323.4 | 500 | 176.6 |
| October | 207.9 | 300 | 92.1 |
| November | 105.6 | 300 | 194.4 |
| December | 89.1 | 300 | 210.9 |
| TOTAL: | 3,676.2 | 4,300 | 1004.8 |

| CAPEX (SPP cost) | 162,877.85 | UAH |
| --- | --- | --- |
| Power of SPP | 3.3 | kW |
| The rate of $ in calculations | 36.65 | UAH |
| Generation SPP by 1 year | 3,676.2 | kW/h |
| Consumption electricity an object | 4,300 | kW/h |
| Online purchase | 1,004.8 | kW/h |
| Coverage of consumption by own SPP | 3,295.2 | kW/h |
| The projected amount of electricity that will not be used from its own SPP | 381 | kW/h |
| Estimated price for 1 kWh from the grid | 6 | UAH |
| Costs for purchasing electricity from the grid after installation of the SPP | 6,028.8 | UAH |
| Costs on service by 1 year | 2,500 | UAH |
| The cost of projected energy consumption from own SPP | 19,771.2 | UAH |
| Cost consumption electricity without installed SPP | 25,800 | UAH |
| Savings of funds after the installation of SPP in the year before full payback | 17,271.2 | UAH |
| Payback of the project | 9.4 | years |
| The cost of generated electricity after payback | 0.68 | UAH per kWh |

PROJECTED PERFORMANCE INDICATORS OF THE PROJECT

SPP 3.3 KW.