| Українська | English |
| --- | --- |
| Сторона В Ми дбаємо про здоров’я врожаю. Основні переваги захисту рослин за допомоги безпілотного літака DR-60 ефективність працює при вологому ґрунті повністю автоматизований політ обприскує в три рази швидше Економічність диференційоване внесення ЗЗР зменшення витрат води у 100 разів економія на пальному більш ніж в 5 разів екологічність не забруднює підземні води не пошкоджує рослини не ущільнює ґрунту Контакти ypederi@aerodrone.biz 0672206943 Сторона А Чим ми займаємось АероДрон – це інноваційна компанія з командою фахівців, що розробляють безпілотні літаки для обприскування у промислових масштабах, які є більш ефективніші за існуючу техніку. Ми впроваджуємо нові технології в точне землеробство, щоб допомогти фермерам збільшити якість та кількість врожаю шляхом захисту рослин за допомогою безпілотних літаків. Послуги з захисту рослин Ми надаємо фермерам послуги з внесення засобів захисту рослин (ЗЗР) методом ультра-малообємного обприскування (УМО) з усіх видів робіт: гербіциди, інсектициди, фунгіциди, десиканти, мікро-добрива). Вартість робіт складає 5-10 у.о. за Га за одну обробку. Замовлення онлайн за посиланням: goo.gl/AAAaaa Особливості експлуатації Для зльоту/посадки достатньо рівної поверхні Д: 150 м та Ш: 15 м. Політ проходить в повністю автоматичному режимі по координатам GPS Насоси розпилюють препарати лише у визначених місцях в межах поля Для експлуатації літака під час роботи в полі, потрібно всього два оператори Технічні характеристики Робоча швидкість Розмах крил Довжина літака Максимальна вага Корисний вантаж Норма внесення Висота польоту Ширина захвату Розмір краплі Продуктивність; 100 км/год 6,5 м 4 м 150 кг 60 кг 1-3 л/Га 5-10 м 20 м 100 мкр 60 Га/год. «Ультра-малооб’ємне обприскування за допомогою безпілотних літаків DR-60 – це нова ера захисту рослин» доктор с/г наук В. А. Санін Метод обприскування Метод УМО значно зменшує витрати робочого розчину. Монодисперсне розпилення утворює однорідну хмару, яка щільно та рівномірно покриває поверхню рослин, а статичний заряд крапель забезпечує краще зчеплення. Високоточне обприскування, в оптимальний час в потрібному місці, дозволяє збільшити врожайність до 50%. | Side В We care about the health of the crop. The major benefits of plant protection with help of airplane DR-60 (automatically flight). Effectiveness Works on wet soil Fully automated flight Spray in 3 times faster Economy Differentiated introduction PPT Reducing water consumption by 100 times fuel economy more than 5 times Ecology Doesn’t pollute underground water Doesn’t damage the plants Doesn’t compress the soil Contacts ypederi@aerodrone.biz 0672206943 Side А What are we doing? AirDron is innovation company with a team of specialists, which develop airplanes with automatically flight for spray in industrial scale, which is more efficient than existing equipment. We are introducing new technologies in precision farming to help farmers increase quality and amount the harvest by way of protecting plants with using unmanned airplanes. Plant protection services We give farmers services with input plants protecting tools (PPT) by method ultra-low spray (ULS) of all types of work: herbicides, insecticides, fungicides, desiccants, micro fertilizers. The cost of works is 5-10 сonditional units per Hectare for one treatment. Online ordering via the link: goo.gl/AAAaaa Features of operation For fly up/landing enough equal ground L: 150 m and W: 15 m. The flight is fully automatic. Airplane use GPS coordinates. Pumps spray the pesticides only in designated areas within the field. To operate an airplane while working in the field, you need only two operators. Specifications Working speed Wingspan Length of the plane Maximum weight Payload Rate of contribution Flight height Width of capture The size of the drop Productivity; 100 km/hour 6,5 m 4 m 150 kg 60 kg 1-3 l/Hectare 5-10 m 20 m 100 mcr 60 Hectare/hour. «Ultra-low spray with unmanned airplanes DR-60 is new era of plants protection» - Doctor of Agricultural Sciences V. A. Sanin. Spraying method Method ULS greatly reduces spending of working pesticides. Monodisperse spraying forms a homogeneous spray cloud, which covers the surface of the plant evenly with high density and a static charge of droplets ensures droplets stick to the plant. High accuracy spraying at the optimal time in the right place allows you to increase the yield to 50%. |