| # NTN-SNR bearing with magtape, produced by ASB technology ### NTN-SNR bearing with magtape, produced by ASB technology ASB (Active Sensor Bearing – a bearing with an active speed sensor). The SNR Company invented and patented this bearing in 1997. For the first time in the world the invention allowed substituting a gear wheel with an inductive sensor by an active sensor and magnetic encoder integrated in a sealing. Patented technology: A wheel end bearing has a sealing with an integrated magnetic encoder in it, with interleaved polarity, over which the sensor is fixed, locking wheel’s spin speed. The signal generated by the magnetic field of alternating polarity and recorded by a sensor, enters the microprocessor unit of a vehicle’s onboard computer for use in car - systems: ABS, ESP, navigation system, help with starting and others. ASB technology is applicable to all types of wheel end bearings. SNR Company has provided the right to use its patent to the world's leading bearing manufacturers (admitted as suppliers of quality products for automotive conveyors), that allowed raising ASB bearing to the rank of international standard, and SNR as the inventor of this technology is present on the overwhelming majority of vehicles sold in Europe. Currently, approximately 90% of vehicles rolling off the European conveyors are equipped with bearings produced by SNR technology. This development has also gained widespread among Japanese automakers. During the exhibition, NTN-SNR managers conducted several seminars for independent vehicle service station mechanics. The seminars were devoted to the nuances of the installation of wheel bearings with magtape. Installation requirements for a bearing with magtape: - keeping cleanness of the workplace and spare parts - installation of a bearing in an axle swivel by a pressing-in method, and the exerted force is only applied on an outer ring of a bearing - seating in a wheel hub by a pressing-in method, and the exerted force is focused on an inner ring of a bearing Additional installation requirements for a bearing with magtape: - 1st generation bearings (product item begins with X ...): to determine the correct position of a bearing: a magnetic ring is integrated from the side of a bearing, where a part’s number is placed. This side should be always turned to the ABS sensor - installation should be performed in such a way as to ensure the protection of a sealing from shock or contamination with metal debris - do not place a bearing near the magnetic field source of the intensity of 750 Gauss (recommendation for mechanics: do not stand a magnetic search engine against ASB magtape) - a bearing with a damaged magnetic encoder (scratches, cracks, demagnetizing) should not be used To ensure proper installation of a bearing with magtape, the Moscow NTN-SNR office offers a special ASB test card if requested. It helps mechanics to determine the location and the intactness of a magtape quickly and accurately for the proper installation of a wheel end bearing. | # Подшипник NTN-SNR с магнитной лентой, выполненный по технологии ASB ### Подшипник NTN-SNR с магнитной лентой, выполненный по технологии ASB (Active Sensor Bearing – подшипник с активным датчиком скорости. Компания SNR изобрела и запатентовала данный подшипник в 1997 году. Данное изобретение позволило впервые в Мире заменить зубчатое колесо с индуктивным датчиком активным сенсором и магнитным энкодером, интегрированным в уплотнение. Запатентованная технология: Ступичный подшипник имеет уплотнение с интегрированным в него магнитным энкодером с чередующейся полярностью, над которым зафиксирован датчик, фиксирующий скорость вращения колеса. Сигнал, создаваемый магнитным полем переменной полярности и регистрируемый датчиком, поступает в микропроцессорный блок бортового компьютера автомобиля для использования в - системах: ABS, ESP, навигационной системе, помощь при трогании с места и др. ASB технология применима для всех типов ступичных подшипников. Компания SNR предоставила право пользования своим патентом ведущим мировым производителям подшипников (признанных как поставщики качественной продукции для автомобильных конвейеров), что позволило возвести ASB подшипники в ранг международного стандарта, а SNR как изобретатель данной технологии по праву присутствует на подавляющем большинстве продаваемых в Европе автомобилей. На сегодняшний день около 90% автомобилей, сходящих с европейских конвейеров, оборудованы подшипниками, произведёнными по технологии SNR. Данная разработка получила также широчайшее распространение среди японских автопроизводителей. Во время выставки менеджеры компании NTN-SNR провели несколько семинаров для механиков независимых СТО . Семинары были посвящены нюансам установки колесных подшипников с магнитной лентой. Требования к установке подшипника с магнитной лентой: - соблюдение чистоты рабочего места и соединяемых деталей - установка подшипника в поворотный кулак способом запрессовки, причем прикладываемое усилие оказывается исключительно на внешнее кольцо подшипника - посадка в ступицу колеса способом запрессовки , причем оказываемое усилие сосредоточено на внутреннем кольце подшипника Дополнительные требования к установке подшипника с магнитной лентой: - Подшипники 1-го поколения (артикул начинается с X...): определить правильное положение подшипника: магнитное кольцо интегрировано со стороны подшипника, на которую нанесён номер детали. Эта сторона должна быть всегда обращена к датчику ABS. - монтаж должен осуществляться таким образом, чтобы обеспечить защиту уплотнения от ударов или загрязнения металлическими частицами - не располагать подшипник вблизи источника магнитного поля напряженностью более 750 Гаусс (рекомендация механикам: не прислонять магнитный поисковик к магнитной ленте ASB) - подшипник с повреждённым магнитным энкодером (царапины, трещины, размагничивание) не подлежит эксплуатации Чтобы удостовериться в правильности установки подшипника с магнитной лентой, московский офис NTN-SNR предлагает по запросу специальную тест карточку ASB. Она помогает механикам быстро и достоверно определить местоположения и исправность магнитной ленты для правильной установки ступичного подшипника. |
| --- | --- |