| Chemical and Pharmaceutical Documentation: II C Control of starting materials | II C3-310800-G1-01 |
| --- | --- |
| II C.3 Packaging material (immediate packaging) | Page 1 of 4 Date 31.08.2000 |

II C.3.1	SPECIFICATIONS AND ROUTINE TESTS

II C.3.1.1 	Type of material

Container:	Amber glass type III

Screw cap A:	HDPE

Screw threaded ring mounted in the screw cap A: 	LLDPE alternative LDPE

Screw cap B:	HDPE

Screw threaded ring with sealing plate in cap B:	LDPE

II C.3.1.2 	Construction

Containers volumes:

30/50/60/75 ml with neck suited for 34 mm cap A and cap В

100/125/175/200/300 ml with neck suited for 42 mm cap A and cap В

II C.3.1.3 	Quality specifications (routine tests) and test procedures

The glass containers are controlled by the manufacturer and analysis certificate is received in connection with the delivery

The 34 mm screw cap A and the 42 mm screw cap A are tested in accordance with the below specifications

| Test | Limit | Method |
| --- | --- | --- |
| Identification of HDPE | Positiv | Melt Index and Density |
| Appearance | No foreign material on the inner surface. Without damage or deformation which may cause problems with tightness, problems on the filling line, or problems for the consumer regarding unscrewing the cap | Visual Inspection Compared to reference screw cap |

| Chemical and Pharmaceutical Documentation: II C Control of starting materials | II C3-310800-G1-01 |
| --- | --- |
| II C.3 Packaging material (immediate packaging) | Page 2 of 4 Date 31.08.2000 |

The 34 mm screw threaded ring in the 34 mm screw cap A and the 42 mm screw threaded ring in the 42 mm screw cap A

| Test | Limit | Method |
| --- | --- | --- |
| Identification of LLDPE alt. LDPE | Positiv | Melt Index and Density |
| Appearance | No foreign material on the inner surface. Without damage or deformation which may cause problems with tightness, problems on the filling line, or problems for the consumer regarding unscrewing the cap | Visual inspection Compared to reference screw threaded ring |

The 34 mm screw cap В and the 42 mm screw cap В are tested in accordance with the below specifications

| Test | Limit | Method |
| --- | --- | --- |
| Identification of HDPE | Positiv | IR Compared to IR spectrum from supplier |
| Appearance | No foreign material on the inner surface. Without damage or deformation which may cause problems with tightness, problems on the filling line, or problems for the consumer regarding unscrewing the cap | Visual inspection Compared to reference screw cap |

The 34 mm screw threaded ring in the 34 mm screw cap В and the 42 mm screw threaded ring in the 42 mm screw cap В

| Test | Limit | Method |
| --- | --- | --- |
| Identification of LDPE* | Positiv | IR Compared to IR spectrum from supplier |
| Appearance | No foreign material on the inner surface. Without damage or deformation which may cause problems with tightness, problems on the filling line, or problems for the consumer regarding unscrewing the cap | Visual inspection Compared to reference screw threaded ring |

As the caps are purchased from a reputable manufacturer and are accompanied

| Chemical and Pharmaceutical Documentation: II C Control of starting materials | II C3-310800-G1-01 |
| --- | --- |
| II C.3 Packaging material (immediate packaging) | Page 3 of 4 Date 31.08.2000 |

by a certificate of analysis demonstrating full compliance with the stated requirements, the full range of tests performed by Nycomed Denmark A/S may be reduced.

II C.3.2	SCIENTIFIC DATA

II C.3.2.1	Materials

| Part of packaging | Material | Quality Standard |
| --- | --- | --- |
| Glass Container | Amber type III glass | Ph.Eur. Glass Containers for Pharmaceutical Use |
| Screw cap A 34 and 42 mm and screw cap В 34 and 42 mm Screw threaded ring in 34 mm screw cap A and 42 mm screw cap A Screw threaded ring in 34 mm screw cap В and 42 mm screw cap В | HDPE dyed with white master batch TiO2 LDPE alternative LLDPE dyed with white master batch TiO2 LDPE dyed with white master batch TiO2 | BgVV (previously BGA); Recommendation III „Polyethylene” and FDA: CFR 21 §117.1520 „Olefin polymers" |

II C.3.2.2	Container

The container is in accordance with Ph.Eur. Glass Containers for Pharmaceutical Use.

II C.3.2.3	Development studies on the packaging material

Tightness of the packaging

Tested according to USP with exception of the opening procedure, the packaging is in compliance with the requirement for tight container.

II C.3.2.4 	Batch analyses

Certificate of analyses for containers and caps to tablet containers shall demonstrate full compliance with the specifications.

| Chemical and Pharmaceutical Documentation: IIC Control of starting materials | II C3-310800-G1-01 |
| --- | --- |
| II C.3 Packaging material (immediate packaging) | Page 4 of 4 Date 31.08.2000 |

Eva Baerentsen

* Not tested routinely