The technical differences between the "home studio", the professional studio and the large sewing industry are predominantly in the volume of production. The same clothing patterns can be created here with the same set of parts and, accordingly, processing operations. And only the quantity of products determines the number of sewing equipment and its features.

What is the difference between domestic and  industrial machines:

- Functions   Modern machines are quite versatile for all types of fabric used in everyday life, but are designed to work with light and medium fabrics (by mass density).

- Operations   Modern household sewing machines have the ability to perform various rows and functions, including . loosening of loops, decorative and knitting stitches, zigzags, fasteners, etc. An industrial sewing machine is designed to perform 1 operation (with a rare exception.

- Speed   sewing on a household machine on average up to 800 stitches per minute. The speed of sewing an industrial machine can reach 8,000 and 12,000 stitches per minute, but more popular machines have a speed of 2000 to 5000 stitches per minute.

- Materials.   Because of the high load, an industrial sewing machine can not have a plastic case, plastic gears, aluminum levers and other similar parts used in the design of household sewing machines.

- Electric drive   domestic machines have a collector (brush) electric drive of not more than 100 watts operating in a repeatedly short-term mode. In modern industrial sewing machines, various electric drives are used: friction, induction, asynchronous, electronic, servo and other drives, having a power from 200 to 1000 Watts and able to work around the clock without stopping.

- Grease   In sewing machines, drip lubrication of knots and aggregates is used, and now the system of "dry" lubrication, calculated for a certain time of trouble-free operation, is becoming increasingly popular. In industrial sewing machines, special lubrication systems are used.

- Operation time.   Modern sewing machines are designed to work for about 7 years, provided that they are used at home for no more than 1 hour a day. Modern industrial sewing machines are designed for work for about 10 years under the condition of round-the-clock application.

The main parameter that is worth paying attention to when choosing is the power of the unit, which determines the puncture force, as well as the type of the shuttle. The basic criteria for selection will be: The material of the body and the parts used, the puncture force will become clear to you, after determining the power of the unit, sewing speed, foot pressure on the fabric surface, hook type, stitch types, “automatic” or “semi-automatic” loop. It is very important to determine which fabric you will work with, since most models can easily cope with light costume and knit fabrics, and you may have problems with working with denser fabrics. Always pay attention to what is included in the package of your chosen model. As a rule, most often it consists of interchangeable legs and a set of needles.
Classification of domestic sewing machines

- Mechanical machine.

- Electromechanical machine.

- Electronic machine.

- Computerized machine

- Sewing and embroidery machine

- Overlockers

We offer you a good selection of machines in our store on the site and there you can buy domestic sewing machine ( https://konsew.com/domestic-sewing-machines ) , in the range of models such as: Bernina Minimatic 807 Sewing Machine with Pedal & HardCase , Janome CoverPro 2000 CPX Coverstitch Machine , BERNINA 807 HEAVY DUTY SEWING MACHINE IN EXCELLENT CONDITION, BERNINA 1015 HEAVY DUTY SEWING MACHINE IN EXCELLENT CONDITION. Domestic sewing machine price between 190 £ till 500 £