Da Vinci’s Robot

Da Vinci robot is a universal surgical system, a robot surgeon, the use of which allows to perform minimally invasive operations, to perform surgical treatment point-by-point, almost without affecting adjacent healthy tissues. Operations with the robot are successfully carried out in leading clinics in Israel, Germany, USA, and more recently in Russia.

Da Vinci's surgical device gives the doctor a better visualization of the process than with an open operation, higher accuracy and control of all manipulations.

This surgical system consists of three main segments: the control panel, behind which the surgeon works, the operating console and the optical system.

The operating console has a special sterile cover and is equipped with four interactive tool manipulators, two of them reproduce the actions of the surgeon and have seven degrees of freedom (more than the brush of a human hand) and can twist and turn at 90 C. the Surgery is performed endoscopically through small incisions of 1 – 2 cm. Extended range of motion improves access and reliability for operations in confined spaces (e.g. in the pelvic area, on the heart sac). The third manipulator serves as an assistant, the fourth is the camera.

The optical system instantly processes the image from the camera and creates a three-dimensional image of the operating field in 3D color format with high resolution. "The eyes of a surgeon" in this case are the two fiberoptic channels – two-lens stereo endoscope and two three-chip cameras. The information is transmitted to two high-resolution color monitors.

The surgeon controls the entire treatment process and the actions of the robot, and during the operation is behind the control panel of the system. During the entire operation, the doctor works while sitting, controls all the tools with the fingertips, as a result, the surgeon is less tired, which reduces the likelihood of errors and allows you to achieve better clinical results.

It is believed that the first attempts to create a surgical design-robot belong to Leonardo da Vinci, the first drawing of a certain robot surgeon was made at the end of the XV century.

Advantages of using Da Vinci Robot:

high precision;

minor blood loss;

the risk of complications and infection during and after surgery is close to zero;

the patient needs less time to be in hospital;

apply smaller size and more delicate instruments – minimum post-operative stitches;

the recovery of the patient and his return to the usual way of life is going faster.

The device is used in many areas of medicine - gynecology, urology, general surgery, cardiology and others.